3GPP TSG-SA3 Meeting #107e AdHoc 	draft_S3-221543-r2

- e-meeting, 27 June - 1 July 2022									 Revision of S3-221543+221445

- Source:	Xiaomi, ZTE

- Title:	33.893: New Key Issue on Direct Communication Security

- Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

SA3 is kindly asked to approve the proposed new key issue to be added in TR 33.893.

# 2	References

[1]	SP-220534	New SID on Security Aspects of Ranging Based Services and Sidelink Positioning

[2]	S3-221537	33.893: Draft Skeleton

# 3	Rationale

The SID on Security Aspects of Ranging Based Services and Sidelink Positioning [1] was approved at SA#96e. The skeleton of the Technical Report for the study (TR 33.893) is proposed in S3-221537 [2]. This pCR intends to propose a new key issue on security of unicast direct communication between the UEs for Ranging/Sidelink Positioning service based on the skeleton of TR 33.893.

# 4	Detailed proposal

*************** Start of the 1st Change ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TR 23.700-86: "Study on Architecture Enhancement to support Ranging based services and sidelink positioning".

[y]	3GPP TS 33.503: "Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[z]	3GPP TS 33.536: "Security aspects of 3GPP support for advanced Vehicle-to-Everything (V2X) services".

*************** Start of the 2nd Change ****************

# 5	Key issues

5.X	Key issue #X: Protection of unicast direct communication between the UEs

### 5.X.1	Key issue details

As per TR 23.700-86 [x], for direct communication between the UEs, the architecture and solutions defined for V2X and ProSe will be reused as much as possible. This provides the basis for reusing the direct communication security defined for ProSe in TS 33.503 [y] and for V2X in TS 33.536 [z] to protect the direct communication for Ranging and SL Positioning services between a reference UE and a target UE or between a reference UE and an assistant UE or between a target UE and an assistant UE.

In solutions #4, #14, #19, and #26 of TR 23.700-86 [x], the control signalling of Ranging/Sidelink Positioning over SR5 using e.g. RSPP (Ranging/Sidelink Positioning Protocol) is established after setting up the PC5 unicast direct communication link. Therefore, the security protection of direct communication over SR5/RSPP can rely on the existing security protection of PC5 direct communication as specified in TS 33.503 [y] and TS 33.536 [z]. When the existing PC5 direct link is reused for Ranging/Sidelink Positioning services, as per clause 5.3.3.1.2.1 in TS 33.536 [z], the root key KNRP shared between the UEs communicating over the PC5 link may be kept when there is no active communication session between the UEs and can be reused for protecting new direct communication sessions between the UEs. This implies that the root key of the existing PC5 direct link (e.g. established for ProSe/V2X service) could possibly be reused for Ranging/Sidelink Positioning services between the same pair of UEs.

However, whether or not an existing root key of PC5 direct link established for ProSe/V2X service is sufficient to protect the control signalling over SR5/RSPP for Ranging/Sidelink Positioning services still need to be investigated.

In addition, whereas some solutions in TR 23.700-86 [x] use PC5-S, other solutions propose to use PC5-U, and hence some control messages, e.g. the messages generated by SPRF or GSSF, may be sent as data traffic over the PC5 link according to TR 23.700-86 [x]. Then whether or not the user plane security of PC5 link can provide the required protection for control messages over SR5/RSPP also needs to be analysed.

### 5.X.2	Security threats

For almost all Ranging/Sidelink Positioning services, the information exchanged during PC5 direct communication between the UEs is location related, which is security/privacy sensitive. However for ProSe/V2X services, the information exchanged during PC5 direct communication between the UEs varies based on the specific service run between the UEs, which could possibly be less security/privacy sensitive and requires lower security level. If the root key of an existing PC5 direct link established for ProSe/V2X service is reused for Ranging/Sidelink Positioning services, the security/privacy protection for the location related information of Ranging/Sidelink Positioning services cannot be ensured in case the root key for ProSe/V2X service is compromised.

When there are multiple Ranging/Sidelink Positioning services running between the UEs, all these Ranging/Sidelink Positioning services will be sharing the same root key. Considering that Ranging/Sidelink Positioning services for different purposes may require different security levels (e.g. a Ranging service for public safety vs a Ranging service for recreation), sharing the same root key will lower the high security level required e.g. for public safety to the same security level required e.g. for recreation service.

When reusing user plane of PC5 unicast link to carry control signalling for Ranging/Sidelink Positioning, the security of PC5 user plane activated for ProSe/V2X services may not support the security requirement of the control signalling for Ranging/SL Positioning services. For example, if the integrity protection of PC5 user plane is not activated, the requirement of integrity protection mandated for control signalling cannot be met.

### 5.X.3	Potential security requirements

The 5G Ranging/SL Positioning system should be able to provide security protection for direct communication between the UEs running or assisting Ranging/SL Positioning services.

The 5G Ranging/SL Positioning system should be able to ensure security isolation between Ranging/SL Positioning services and ProSe/V2X services when reusing the security mechanism for ProSe/V2X direct communication.

The 5G Ranging/SL Positioning system should be able to ensure security isolation among the direct communications established between the UEs for various Ranging/SL Positioning services requiring different security levels.

The 5G Ranging/SL Positioning system should be able to ensure that the security activation of PC5 unicast link supports the security requirements of the information carried during the direct communication between the UEs for Ranging/SL positioning services.

*************** End of the Changes ****************