3GPP TSG-SA3 Meeting #107Ad-Hoc-e 	S3-221571-r2

- e-meeting, 27 June – July 1st 2022

- Source:	Lenovo

- Title:	AKMA roaming and LI

- Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

Solution for AKMA roaming.

# 2	References

[1] 3GPP TR 33.737, Study on Authentication and Key Management for Applications (AKMA) phase 2; (Release 18)

# 3	Rationale

The document introduces a solution for AKMA roaming under consideration of the LI requirement.

# 4	Detailed proposal

Start of changes

## 6.Y	Solution #Y: AKMA roaming with VAAnF for LI

### 6.Y.1	Introduction

This solution is addressing Key Issue #1: Support for AKMA roaming scenario, especially the scanrio when the UE is in VPLMN and trying to access the HPLMN AF.

The issue of LI for AKMA roaming was described as follows: if the UE is roaming in a VPLMN, then the UE builds up a secure tunnel to an AF in the HPLMN and since the credentials used for the encryption are based on the 3GPP derived keys, the VPLMN must be able to perform LI. This is not possible compared to GBA, where the NAF and tunnel endpoint is located in the VPLMN.

Further it cannot be implied that the AF is always in the VPLMN for roaming scenarios, for typical deployments it can be a 3rd party AF in a data network.

If the VPLMN needs to perform LI, then the VPLMN needs to be enhanced to store the SUPI and the encryption key e.g. with a local AAnF. It is recommended to further also only provide the KAF to the VPLMN for the service the UE is currently requesting from the AF. In case the VPLMN is not enhanced but has a strong LI requirement for AKMA, the AF should not get the KAF and should get an indication that NULL encryption has to be used.

### 6.Y.2	Solution details

The solution proposes to introduce a VAAnF in the VPLMN in order to store the connection details of the UE roaming in that VPLMN to the AF outside that VPLMN. The differences to the normal procedures of TS 33.535 are described below.

Figure 6.Y.2-1: Deriving KAKMA after primary authentication

Additionally to the other AKMA related parameters, the AUSF provides also the SN name to the AAnF in the HPLMN in step 4. The SN name is later used to determine whether the Ue is roaming and to select an appropriate AAnF in the VPLMN (VAAnF) for storing the AKMA connection details.

Figure 6.Y.2-2: KAF generation from KAKMA and provisioning to VPLMN

In step 3, the HAAnF derives the AKMA Application Key (KAF) from KAKMA if it does not already have KAF.

In step 4, the HAAnF detects based on the SN name that the UE is roaming and if the VPLMN has AKMA LI enhancements, then the HAAnF provides the KAF and the KAF expiration time together with the SUPI of the UE to the VAAnF in the VPLMN for storing the AKMA LI context. The VPLMN AKMA capabilities and policies may be configured in the HAAnF and may be based on SLAs.

Editor’s Note: How to address the case that a VPLMN operator does not support AKMA and is required to perform LI is FFS

Editor’s Note: LI evaluation is FFS

### 6.Y.3	Evaluation

Editor’s Note: Each solution should motivate how the potential security requirements of the key issues being addressed are fulfilled.

End of changes