3GPP TSG-SA3 Meeting #107adhoc-e 	S3-221465

# e-meeting, 27 June – 1 July 2022

# Source:	Apple

# Title:	Adding a key issue of security risk for C-RNTI

# Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

It is proposed to add a new key issue for TR 33.870.

# 2	References

Null

# 3	Rationale

In NR, C-RNTI is the unique UE identification used as the identifier of the rRC Connection and for UE dedicated scheduling [x].

During the initial access procedure from RRC_IDLE/RRC_INACTIVE to RRC_CONNECTED state, the C-RNTI is allocated by NW via the random access procedure, i.e. NW will provide the T-CRNTI in Msg2 and the T-C-RNTI will be the C-RNTI upon the random access procedure is successfully completion.

During the handover procedure in RRC_CONNECTED state, network will provide the new C-RNTI for the target cell to the UE via the handover command (e.g. RRCReconfiguration), and UE will use this C-RNTI in the target cell after the handover.

This will cause the Location tracking [m] and Identity Mapping attack. The C-RNTI assigned to the UE as part of the Random-Access procedure within the RRC Connection Establishment is not secured and can be identified by the Attackers. This enables attackers to map the TMSI “Temporary Mobile Subscriber Identity” received in Random Access procedure Msg3 & Msg4 (containing Random Access Contention Resolution Identity) to the C-RNTI assigned in Random Access procedure Msg2 (RAR). Accordingly, this is enabling the attackers to track data streams addressed to certain TMSI within a cell and possibly attacking them, as described in [y]. Possible attacks from mapping of UE identity “TMSI” to a Mobile Phone number is described in [z].

This vulnerability may be leveraged in the future by attackers to identify this mapping and accordingly track & attack a specific UE communication with the gNB.

This threat is also applied for LTE [y]. Since the TR 33.870 is for NR, how LTE vulnerability should be addressed is not in the scope of this spec.

# 4	Detailed proposal

****START OF CHANGES ***

# 5	Key issues

## 5.X	Key issue #X:

### 5.X.1	Key issue details

In NR, C-RNTI is the unique UE identification used as the identifier of the rRC Connection and for UE dedicated scheduling [x].

During the initial access procedure from RRC_IDLE/RRC_INACTIVE to RRC_CONNECTED state, the C-RNTI is allocated by NW via the random access procedure, i.e. NW will provide the T-CRNTI in Msg2 and the T-C-RNTI will be the C-RNTI upon the random access procedure is successfully completion.

The C-RNTI will be updated in the handover procedure. During the handover procedure in RRC_CONNECTED state, network will provide the new C-RNTI for the target cell to the UE via the handover command (e.g. RRCReconfiguration), and UE will use this C-RNTI in the target cell after the handover.

### 5.X.2	Threats

1) Location Tracking: A passive attacker could monitor the radio channel and link the IMSI with the various C-RNTIs assigned by the eNB/gNB to the UE, so that the attack could keep a user profile history area and could record the locations visited by the target user [m].

2) Identity Mapping: The C-RNTI assigned to the UE as part of the Random-Access procedure within the RRC Connection Establishment is not secured and can be identified by the Attackers. This enables attackers to map the TMSI “Temporary Mobile Subscriber Identity” received in Random Access procedure Msg3 & Msg4 (containing Random Access Contention Resolution Identity) to the C-RNTI assigned in Random Access procedure Msg2 (RAR). Accordingly, this is enabling the attackers to track data streams addressed to certain TMSI within a cell and possibly attacking them, as described in [y]. Possible attacks from mapping of UE identity “TMSI” to a Mobile Phone number is described in [z]. This vulnerability may be leveraged in the future by attackers to identify this mapping and accordingly track & attack a specific UE communication with the gNB.

NOTE: This threat is also applied for LTE [y]. How to address the LTE vulnerability is out of the scope of this spec.

### 5.X.3	Potential security requirements

5G System should provide the support for the Identity Mapping attack caused by the unprotection of C-RNTI.

****START OF Second CHANGES ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[y]	Breaking LTE on Layer Two, https://alter-attack.net/media/breaking_lte_on_layer_two.pdf

[z]	A. Shaik, R. Borgaonkar, N. Asokan, V. Niemi, and J.-P. Seifert, “Practical Attacks Against Privacy and Availability in 4G/LTE Mobile Communication Systems,” in Symposium on Network and Distributed System Security (NDSS). ISOC, 2016.

[m]	Mamoon M. Saeed, Mohammad Kamrul Hasan, Rashid A. Saeed4 Md Akhtaruzzaman, Ahmed J. Obaid, Rania A. Mokhtar,  Elmustafa Sayed Ali, Sanaz Amanlou, A. K. M. Zakir Hossain, A comprehensive review on the users’ identity privacy for 5G networks, https://ietresearch.onlinelibrary.wiley.com/doi/pdf/10.1049/cmu2.12327

****END OF CHANGES ***