3GPP TSG-SA3 Meeting #107Adhoc-e	S3-221480

# e-meeting, 27th June – 1st July, 2022

# Source:	OPPO

# Title:	New solution: Security procedures of KAF refresh using counter via 5GC

# Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

This pCR proposes to solve the Key Issue #2 in TR 33.741[1]

# 2	References

[1] 	3GPP TS 33.741 " Study on home network triggered primary authentication (HONTRA)".

# 3	Rationale

This contribution addresses the security requirements for key issue #2 in TR 33.741[1].

# 4	Detailed proposal

SA3 is kindly requested to agree to the below pCR to TR 33.741 [1].

**** START OF CHANGE ****

## 6.X	Security procedures of KAF refresh using counter via 5GC

### X.X.1	Introduction

This solution addresses the new KAF generation after the lifetime of KAF expires, while the UE has not run a new primary authentication, which also avoid the AF inappropriate reject UE’s access after the KAF expires. Specifically, UE and AAnF can maintain a counter to derive a new KAF* from the expired KAF. The counter is specific to the KAF as part of the AMKA context. Counter always starts with value 0 for a new KAF derived from KAKMA and is incremented by 1 with each KAF refresh.

### X.X.2	Solution details

Figure 6.X.1: Security procedures of KAF refresh using counter via 5GC

1. UE initiates communication with the specific AF, and include the latest A-KID in the Application Session Establish request.

2a. AF retrieves the AKMA context using the A-KID, and find out the lifetime of the KAF associated with the A-KID has expired.

2b. AF then sends Naanf_AKMA_ApplicationKey_Get request to AAnF to request the KAF refresh, which contains A-KID, AF_ID, and KAFRefreshInd indicating the KAF refresh.

3a. AAnF retrieves the UE AKMA context according to the received A-KID. According to the counter associated with the KAF, AAnF increases the counter by one and derives KAF* according to clause 6.X.2.2.

3b. AAnF sends KAFKey_Generation Request to the UE though AUSF, which contains A-KID, counter.

3c. AAnF sends the Naanf_AKMA_ApplicationKey_Get response to AF, which contains KAF*, KAF* expiration time, SUPI (if needed).

4. UE derives KAF* according to clause 6.X.2.2.

5. AF sends the UE Application Session Establishment Response message.

#### 6.X.2.2	Derivation of KAF*

When deriving a KAF* from KAF, the following parameters shall be used to form the input S to the KDF:

-	FC = 0xXX;

-  P0 = AF-ID;

-  L0 = length of AF-ID;

-  P1 = Counter;

-  L1 = length of Counter.

The input key shall be KAF.

### X.X.3	Evaluation

This solution addresses KAF refresh without having to run a new primary authentication between UE and the network.

AAnF impact: AAnF needs to maintain the KAF counter. AAnF needs to support an additional message sent to UE (via AUSF) for KAF Key_Generation request.

UE impact: UE needs to compute new KAF when it receives the KAF Key_Generation request message from AAnF via AUSF.

**** END OF CHANGE ****