3GPP TSG-SA3 Meeting #107-e 	S3-220708

- e-meeting, 16 - 20 May 2022

- Source:	China telecom

- Title:	New KI: Home network triggered primary authentication

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

It is requested to approve this new key issue on home network triggered primary authentication of TR 33.741.

# 2	References

[1]	3GPP TS 33.741 Security aspects of home network triggered re-authentication.

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[3]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS) ".

# 3	Rationale

Home network triggered primary authentication is often required, for example, UPU/SoR COUNT wrap around, refresh of KAUSF. This contribution adds a key issue in order to study this requirement.

# 4	Detailed proposal

**** START OF CHANGE 1 ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[XX]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[XX]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS) ".

**** END OF CHANGE 1 ****

**** START OF CHANGE 2 ****

- 5.X	Key Issue #X: Home network triggered primary authentication

- 5.X.1	Key issue details

This key issue shows some situations where home networks need to trigger primary authentication, as follows:

- If COUNT wrap around happens, then the UPU/SoR service will be suspended as specified in TS 33.501. When a fresh KAUSF is generated for the UE based on primary authentication run, the Counter UPU /SoR at the AUSF is reset to 0x00 0x01. At this time, the AUSF shall resume the UPU/SoR protection service for the UE.

- KAKMA shall be derived from KAUSF as specified in TS 33.535. Since KAKMA and A-TID in A-KID are both derived from KAUSF based on primary authentication run, the KAKMA and A-KID can only be refreshed by a new successful primary authentication. When the KAF lifetime expires and the KAKMA has not changed in AAnF, the KAF which is established based on the current KAKMA is not a new one.

Therefore, in order to meet the above requirements, it is necessary that home network trigger primary authentication.

- 5.X.2	Security threats

TBD

- 5.X.3	Potential security requirements

Home network should be able to trigger primary authentication.

**** END OF CHANGE 2 ****