- 3GPP TSG SA WG3 Meeting #107-e                            	S3-220824

- e-meeting, 16-20 May 2022	                         revision S3-22abcd

- Source:	Huawei, HiSilicon

- Title:	New KI for Security protection of NF certificate enrolment

- Document for:	Approval

Agenda Item:	5.7

# Decision/action requested

This contribution proposes a new key issue in TR 33.876

# References

# Rational

An instantiated NF needs to obtain the certificate from the CA for communication with other NFs. To ensure the security of communication between NFs, a secure and automated certificate enrolment procedure is indispensable.

# Detailed proposal

*************** Start of 1st Change ****************

## 5.X	Key Issue #X: Security protection of NF certificate enrolment

### 5.X.1	Key issue details

### An instantiated NF needs to obtain the certificate from the CA for communication with other NFs. To ensure the security of communication between NFs, a secure and automated certificate enrolment procedure is indispensable.

This key issue consider the procedure of certificate enrolment, the protection of certificate enrolment procedure and the authentication between NF and CA.

### 5.X.2	Security threats

If the NF cannot obtain a certificate, and the certificate enrolment procedure is not secured, the following problems may occur:

- The NF cannot communicate with each other.

- If certificate enrolment parameters are tampered, the CA issues an incorrect certificate.

- Without pre-established trust between the NF and CA/RA, an attacker claiming to be a trusted NF with connectivity in SBA could apply for a valid operator certificate.

- 5.x.3	Potential security requirements

5GS should support the means to secure the automated enrolment of certificates, include integrity protection and Anti-replay protection of enrolment procedure.

5GS should support the mutual authentication between involved parties during the enrolment procedure.

5GS can support the Verifying of certificate validity for certificate enrolment.

*************** End of 1st Change ****************