3GPP TSG-SA3 Meeting #107-e 	draft_S3-220748-r1

- e-meeting, 16 - 20 May 2022

- Source:	ZTE

- Title:	Update the clause 7.4.2

- Document for:	Approval

Agenda Item:	4.7

# 1	Decision/action requested

This contribution proposes to Update the clause 7.4.2.

# 2	References

[1]	3GPP TS 33.503 "Security aspects of enhancement for proximity based services in the 5G System".

# 3	Rationale

This contribution proposes to update the clause 7.4.2. The parameter SN name is missing.

# 4	Detailed proposal

**** 1st CHANGE ****

### 7.4.2	Nudm_UEAuthentication Service

#### 7.4.2.1	Nudm_UEAuthentication_GetProseAv service operation

Service operation name: Nudm_UEAuthentication_GetProseAv

Description: Requester NF gets the authentication data for Prose from UDM. If SUCI is included, this service operation returns the SUPI.

Inputs, Required: SUPI or SUCI, Relay Service Code, Serving network name.

Inputs, Optional: Synchronization Failure indication and related information (i.e. RAND/AUTS).

Outputs, Required: Authentication Vector for Prose.

Outputs, Optional: SUPI if SUCI was used as input.

**** END OF CHANGES ****