3GPP TSG-SA3 Meeting #107-e 	S3-221011-r2

- e-meeting, 16 – 20 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1401 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of inconsistency in SUCI usage during UE onboarding. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2022-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS 33.501 section 5.8.2 and 6.12.5 the location and requirements concerning the deconcealment function transforming the SUCI to a SUPI is defined. These sections specify, that the SUCI shall be deconcealed as part of the UDM. According to the text in annex I, onboarding using a DCS omits the interaction with the UDM and hereby the capability to deconceal the identifier. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR proposes to restrict the use of EAP-AKA’ and 5G-AKA to usage where the DSC holds an AUSF/UDM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not accepted, onboarding of UE’s using a DCS acting as a AAA will disclose the identity when EAP-AKA’ or 5G-AKA is used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex I.9.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGES ****

## I.9.2	Authentication

### I.9.2.1	Requirements

The primary authentication shall be performed before initial access for UE onboarding is allowed. The UE shall use Default UE credentials for the primary authentication. Credentials or means used to authenticate the UE based on Default UE credentials may be stored within the ON-SNPN or in a Default Credentials Server (DCS) that is external to the ON-SNPN.

Editor’s Note: It is FFS how using anonymous SUCI or skipping default credentials identifier to initiate onboarding will meet the scope of ‘UE being verified as "uniquely identifiable and verifiably secure".

Editor’s Note: It is FFS, how the default credential identifier i.e., verifiably secure identifier is used as SUPI during the authentication procedure related to Onboarding.

### I.9.2.2	Primary authentication without using DCS

When the primary authentication is performed between the UE and the ON-SNPN, any one of the existing authentication methods defined in the present document may be used, i.e., 5G AKA, EAP-AKA’ or any other key-generating EAP authentication method (e.g., EAP-TLS).

The choice of primary authentication method used is left to the decision of the ON-SNPN.

Credentials required to authenticate the UE using default UE credentials, are provisioned at the AUSF or AUSF/UDM of the ON-SNPN. The provisioning of this information is out of scope of this document.

### I.9.2.3	Primary authentication using DCS

When the primary authentication is performed between the UE and the DCS, the authentication requirements and procedures defined in clause I.2 for Credential Holder shall apply with the DCS taking the role of the Credentials Holder. When the DCS uses AAA Server for primary authentication, AUSF directly selects the NSSAAF as specified in 23.501 [2]. In this case, the UDM is not involved in the procedure defined in clause I.2.2.2.2, and the step 3 to step 5 shall be skipped.

When 5G AKA or EAP-AKA’ is used, the DCS shall act as a AUSF/UDM.

The choice of primary authentication method used between the UE and the DCS is left to the decision of the DCS.

When the primary authentication is performed between the UE and the DCS via the AUSF using EAP-TTLS, Annex U can be used.

****** END OF CHANGES ****