- 3GPP TSG-SA3 Meeting #107-e 	draft_S3-221096-r1

- e-meeting, 16 – 20 May 2022

- Source:	Huawei, Hisilicon

- Title:		Update of Solution #12

- Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

This contribution proposes to update of the solution #12 by removing the EN and adding the evaluation.

# 2	References

[1]	3GPP TR 33.875 Study on enhanced security aspects of the 5G Service Based Architecture (SBA); (Release 17)

# 3	Rationale

Quote for TS 33.310 clause 6.1.3c SBA Certificate profile:

…

-	subjectAltName should (in TLS server certificates) contain URI-IDs  with the HTTPS URI(s) for the apiRoot of a Network Function consumer instance for the NF service callback URI(s) that it provides; using wildcard URIs should be avoided;

…

The callback URI(s) is already captured in its certificate.

Hence it is proposed to use the certification the NF_C or the NF profile of the NF_C to verify the notification URI of NF_C. Then, the following EN can be removed, i.e.

Editor’s Note: It is ffs how the NRF determines whether the notification URI of the NF_C is authorized to receive the requested service from NF_B, since not all consumers register a profile at the NRF and the callback URI could be allocated dynamically for implementation specific reasons.

Futhermore, the evaluation is added.

# 4	Detailed proposal

******************Start of the change*****************

## 6.12	Solution #12: Authorization of notification endpoint in “Subscribe-Notify” scenarios

### 6.12.1	Introduction

This solution addresses key issue #3: Service access authorization in the “Subscribe-Notify” scenarios by including “notification URI” in the token request.

This solution proposes to include “notification URI” and CCA of the NF which owns the “notification URI” at the access token request. The NRF verifies whether the notification URI is authorized to receive the service/notification that is requested in the access token request.

### 6.12.2 	Solution details

During access token request process, NF Service Consumer include "notification URI" in Nnrf_Access Token_Get Request when operation semantics of the requested service type is "Subscribe/Notify". NRF may verify whether the notification URI in the access token request match the corresponding information in the public key certificate of the NF Service Consumer or those in the NF profile of the NF Service Consumer. In addition, when "notification URI"(s) are included in the access token request, NRF may verify whether the "notification URI"(s) locate its location(s) with the same address(es) which are indicated by its own FQDN or IP address as registered in NRF. The NRF checks whether the NF Service Consumer is authorized to access the requested service(s).

When the NF Service Consumer is authorized, the NRF shall then generate an access token with appropriate claims which may include “notification URI” as requested by NF Service Consumer.

Figure 6.12.2.-1: Access token request for "Subscribe-Notify" NF Service illustration 1

A NF Service Consumer (NF_A) may request to subscribe to NF Service offered by a NF Service Producer(NF_B) on behalf of NF_C according to the Service Request from NF_C (e.g. Figure 4.15.3.2.2-1: Nudm_EventExposure_Subscribe, Unsubscribe and Notify operation in TS23.502).

The Figure 6.12.2-2 describe the solution to verify the service request from NF_A including Notification URI of NF_C.

Figure 6.12.2-2: Access token request for "Subscribe-Notify" NF Service illustration 2

Step 0.	The NF_C sends a Service Request to the NF_A for notification service on an event with an access token including notification URI and CCA of the NF_C.

Step 1.	The NF_A decides to subscribe a service of the NF_B associated to the Service Request received at step 0 and sends an Access Token Request to the NRF for a Service Request toward the NF_B including notification URI and CCA of the NF_C.

Step 2.	The NRF verifies whether the NF_A is authorized and whether the NF_C identified by the appended CCA is a valid Network Function and authorized to receive the requested NF Service from the NF_B. NRF may verify whether the notification URI in the access token request match the corresponding information in the public key certificate of the NF_C of the CCA of the NF_C, or those in the NF profile of the NF_C.

Step 3.	Upon successful verification, the NRF publishes an access token for a Service Request of the NF_A toward the NF_B. The access token includes notification URI of the NF_C in the claims as requested in step 2.

Step 4.	The NF_A sends a Service Request for a notification service toward NF_B including the access token received at step 3 and CCA of the NF_A.

Step 5.	The NF_B verifies the access token and CCA of the NF_A. During the verification, the NF_B can check whether notification URI in the Service Request are included in the access token. If the verification is successful, the NF_B may respond the NF_A and provide the NF_C with the notification service as requested by the NF_A.

******************Next change*****************

### 6.12.3 	Evaluation

This solution addresses the threats and requirements of Key issue #3.

This solution provides an approach to solve the notification URI authorization by using the certificate of the NFc or its profile, in order to prevent information disclosure to an unauthorized NF.

******************End of the change*****************