3GPP TSG-SA3 Meeting #107-e 	S3-220740r2

- e-meeting, 16 - 20 May 2022

- Source:	China Telecom

- Title:	New KI on authorization of selection of participant NWDAF instances in the Federated Learning group

- Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

This contribution proposes to add new KI in supporting Federated Learning in the 5GC.

# 2	References

[1]	3GPP TR 23.700-81 Study of Enablers for Network Automation for 5G System (5GS); Phase 3

# 3	Rationale

The contribution proposes to add new KI in supporting Federated Learning in the 5GC.

# 4	Detailed proposal

It is suggested to approve the following change.

*************** Start of the  change ****************

### 5.x	Key Issue #x: Authorization of selection of participant NWDAF instances in the Federated Learning group

### 5.x.1	Key issue details

3GPP SA2 studies the architecture enhancement to support Federated Learning which allows the cooperation of multiple NWDAFs containing MTLF to train an ML model in 3GPP network. As per KI#8 in TR 23.700-81, it will involve selection of participant NWDAF instances in the Federated Learning group.

This key issue studies the authorization aspect of including participant NWDAF instances in the Federated Learning group.

### 5.x.2	Security threats

If a Federated Learning group can include a NWDAF without being authorized by the NWDAF, it may lead to the following issues:

- NWDAF(MTLF)’s resource may be used up by being included into many unauthorizd Federated Learning groups.

- Sensitive data may be used to train unauthorizd Federated Learning group’s ML model.

- Unauthorizd Federated Learning group may utilize the local model received from NWDAF(MTLF) to infer sensitive training data details.

### 5.x.3	Potential security requirements

Authorization of selection of participant NWDAF instances in the Federated Learning group should be supported. A Federated Learning group should not be able to select participant NWDAF instances without NWDAF's authorization.

.

*************** End of the change ****************