3GPP TSG-SA3 Meeting #108e AdHoc	S3-222605r1

- e-meeting, 10 – 14 October 2022

- Source:	OPPO, XIDIAN

- Title:	Key issue on secure provisioning of external parameter

- Document for:	Approval

Agenda Item:	5.20

- 1	Decision/action requested

It is requested to approve the new KI on secure provisioning of external parameters.

- 2	References

[X]		S3-221188 "New SID on Security and Privacy of AI/ML-based services and applications in 5G"

[Y]		3GPP TR 23700-80 "Study on 5G System Support for AI/ML-based Services"

- 3	Rationale

This contribution introduces a new KI in the SID on Security and Privacy of AI/ML-based services and applications in 5G.

It is related to objective #4 in SID S3-221188 [1] which is defined as

4.   Secure provisioning of the external parameter required for AI/ML (e.g., expected UE activity behaviors, expected UE mobility , etc.) -

- 4	Detailed proposal

**** START OF CHANGE ****

- 6.X	KI #X: secure provisioning of external parameters

- 6.X.1	Key issue details

It is related to objective #4.

As outlined in clause 4.15.6 of TS 23.502 [4], provisioning capability allows AF to provision specific parameters, such as expected UE behaviour and other service specific parameters to 5G network functions to assist AF to perform application AI/ML operations. For example, the expected UE behavioural information consists of information on expected UE movement and communication characteristics. Provisioned data may also be used by the other NFs.

The AI/ML model, intermediate data, inference results, or model performance data at the UE are transported by 5GS for the Application AI/ML operation. In order to assist the Application AI/ML operation, the 5GC can maintain and use the external parameters provisioned by AF.

To ensure the security and privacy of UE-related data in external parameters, it is necessary to consider how AF can securely provision external parameters to 5GC.

- 6.X.2	Security threats

If provisioning of external parameter to 5GC network function is not protected, 5GC may not be able to provide the assistance needed by AF to perform application AI/ML operations, for example, the UEs in targeted AOI may not be properly selected or UEs from non-targeted AOI may be incorrectly selected for federated learning.

- 6.X.3	Potential requirements

3GPP system shall support secure provision of AF’s external parameter required for application AI/ML operations.

**** END OF CHANGE ****