- 3GPP TSG-SA3 Meeting #108-Ad Hoc-e	S3-222649-r1

- e-meeting,  10 - 14 October 2022

- Source:	ZTE

- Title:	new KI on the Security of Network Slice Service continuity

- Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

This document proposes to add a new key issue #4: Security of Network Slice Service continuity. SA3 is kindly requested to approve this doc.

# 2	References

[1]	3GPP TR 23.700-41 v1.0.0

[2]	3GPP TR 33.866 v0.1.0

[3]	Bellare, Mihir, and Sara K. Miner. "A forward-secure digital signature scheme." Advances in Cryptology—Crypto’99. Springer Berlin Heidelberg, 1999.

# 3	Rationale

Related issues has been studied in TR 23.700-41 [1] to support network slice service continuity in both no mobility scenario and inter RA mobility scenario. Network slice is changed here, so the security issue should be considered, for example the forward/backward security of the keys should be guaranteed.

Forward/backward security means that after the current key is obtained by an attacker, the future/historical key is still secure [3]. Since the two different slices use the same key, forward/backward security of the key may not be guaranteed.

So we propose to add this new key issue on the Security of Network Slice Service continuity.

# 4	Detailed proposal

**** START OF CHANGE ****

## 4.X	Key Issue #X: Security of Network Slice Service continuity

### 4.X.1	Key issue details

Related issues has been studied in TR 23.700-41 [3] to support network slice service continuity in both no mobility scenario and inter RA mobility scenario. In the solutions, an alternative slice may be used to provide the service continuity of ongoing PDU sessions .However, security issue is not considered and may cause problems. For example, the forward/backward security of the keys should be guaranteed.

Forward/backward security means that after the current key is obtained by an attacker, the future/historical key is still secure. Since the two different slices use the same key in the solutions, the attacker can access the resource of the alternative slice without permission by retrieving the security context of the old slice, or vice versa.

### 4.X.2	Security threats

It may violate the forward/backward security of the keys when the security context keeps unchanged during providing service continuity of different network slices .

### 4.X.3	Potential security requirements

Editor’s note: Further requirements are FFS.

**** END OF CHANGE ****