3GPP TSG-SA3 Meeting #108e 	S3-221798

e-meeting, 22 - 26 August 2022

Title:	LS on alignment of NSACF notification service

Response to:

Release:	Release 17

Work Item:	Enhanced security for Phase 2 network slicing (eNS2_SEC)

Source:	SA3

To:	SA2

Cc:	CT1, CT4

Contact person:	lei.zhongding@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

In TS33.501 (clause 16.6.3 Subscription/unsubscription procedure of NSACF notification service), an S-NSSAI shall not be sent to an AF deployed outside the 3GPP operator domain (i.e. “untrusted” in the SA2’s terminologies) to avoid exposure of the S-NSSAI to the AF. The ENSI (i.e. external network slice information) mapped from S-NSSAI is used instead (e.g. sent as the Event filter parameter). It is notable that ENSI has already been used in the TS33.501 for the NSSAA procedure (Rel-16 and Rel-17) instead of S-NSSAI when sending messages to AAA-S deployed outside the 3GPP operator domain for the same reason.

It has been brought to our attention that the usage of ENSI is not aligned to TS23.502. In TS23.502 specifically,

- In the NSSAA procedure (Rel-16 and Rel-17), S-NSSAI is not protected when being sent to an AAA-S deployed outside the 3GPP operator domain (untrusted)

- In the NSACF procedure (Rel-17), S-NSSAI is mapped to AF-Service-Identifier for the protection of S-NSSAI and the AF-Service-Identifier instead of the ENSI is sent to an AF deployed outside the 3GPP domain (untrusted). In addition, it is noted that AF-Service-Identifier may not be able to uniquely identify a network slice.

In order to align TS33.501 and TS23.502, SA3 proposes changes to TS23.502 as follows:

- no change for the NSSAA procedure in TS23.502

- use ENSI instead of AF-Service-Identifier in the NSACF procedure in TS23.502

This can minimize standard impact and stage 3 specifications can follow TS33.501 for the protection of S-NSSAI.

SA3 is kindly request SA2 to consider the above misalignment issue and update respective specifications or provide feedbacks otherwise.

# 2	Actions

To SA2

ACTION: 	SA3 kindly requests SA2 to take the above information into account in updating respective specifications or providing feedbacks.

# 3	Dates of next TSG SA WG 3 meetings

SA3#108e-Bis	10-14 October 2022	Online

SA3#109e	14 - 18 November 2022	Canada (TBC)