3GPP TSG-SA3 Meeting #108e 	S3-221869

# e-meeting, 22 - 26 August 2022

# Source:	Nokia, Nokia Shanghai Bell

# Title:	KI8 conclusion on deploying multiple NRFs

# Document for:	Approval

Agenda Item:	5.24

# 1	Decision/action requested

Conclusion on key issue 8 is provided.

# 2	References

[1]	3GPP TR 33.875

# 3	Rationale

The key issue was created to document the different deployment scenarios for NRFs. It is concluded to add a clause in line with below analysis to TS 33.501.

# 4	Detailed proposal

## ******** START OF CHANGES

*** Clause 7 on Conclusions

## 7.8	KI#8: Service access authorization requirements in intra-PLMN scenarios for PLMN deploying multiple NRFs (in OAuth2.0 AS role)

### 7.8.1	Analysis

As described in clause 6.2.6.1 of TS 23.501 [1], an operator network can deploy multiple NRFs, for example due to network slicing or network segmentation.

A clause on handling access token requests in deployments with several NRFs is missing in 3GPP TS 33.501 [2]. One solution for this key issue is described as solution #10.

If an NRF receives an access token request for an NF Service Producer that is not registered at this NRF, the NRF has to determine the target NRF where the NF Service Producer is registered. Only afterwards it can forward the access token request to the target NRF.

There can also be several hops of NRFs between the NRF that receives the access token request from the NF Service Consumer and the target NRF where the NF Service Producer is registered.

The solution addresses also the scenario of hierarchical NRF deployment. One option is the local NRF deployment, where the local NRF checks if the NF Service Consumer is authorized to receive the requested service and, if yes, issues and signs the access token. In the case when the access token request from the NF Service Consumer was forwarded by another NRF, the local NRF of the NF Service Producer needs to trust the NRF which forwarded the access token request.

### 7.8.2	Conclusion

A new clause to TS 33.501 in line with above analysis will be added.

## ******** END OF CHANGES