3GPP TSG-SA3 Meeting #108-e 	S3-222096

# e-meeting, 22 - 26 August 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.503 | CR | 0030 | rev | <Rev#> | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Nausf_UEAuthentication_Authenticate service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2022-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For output of Nausf_UEAuthentication_Authenticate service, it defines as followed. Output, Required: EAP message, Authentication result and if success KNR_ProSe, Nonce_2 and 5GPRUK ID. However, when the ProSe authentication is performed, the output includes EAP message or KNR_ProSe, Nonce_2 and 5GPRUK ID when the authentication is success. When the ProSe authentication is not performed, EAP message is not involved. For different scenarios, the output is different, not all the information is required in every response service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add different options in the output of Nausf_UEAuthentication_Authenticate service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect service is defined which may cause interoperation issue. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

7.3.2.1	Nausf_UEAuthentication_ProseAuthenticate service operation

Service operation name: Nausf_UEAuthentication_ProseAuthenticate.

Description: Authenticate the 5G ProSe Remote UE and provides Prose related keying material.

Input, Required: One of the options below:

1)	In the initial authentication request: SUCI or 5GPRUK ID of the 5G ProSe Remote UE, Relay Service Code, Nonce_1.

2)	In the subsequent authentication requests: EAP message.

Input, Optional: None.

Output, Required: One of the options below.

- EAP message.

- Authentication result and if success KNR_ProSe, Nonce_2 and 5GPRUK ID.

Output, Optional: None.

# * * * * End of changes * * * *