3GPP TSG-SA3 Meeting #108-e 	S3-222107

# e-meeting, 22nd – 26th August, 2022                                        		Revision of S3-22xxxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | draft-CR | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update requirement and add new test case to clause 4.2.3.4.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph2 |  |  |  |  |  | Date: |  |  | 2022-08-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some requirement in 4.2.3.4.3.2 is not tested, so the test case is added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add test case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Untested requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of 1st Change ****************.

##### 4.2.3.4.3.2	Password changes

Requirement Name: tba

Requirement Description:

If a password is used as an authentication attribute, then the system shall offer a function that enables a user to change his password at any time. When an external centralized system for user authentication is used it is possible to redirect or implement this function on this system.

Password change shall be enforced after initial login.

The system shall enforce password change based on password management policy. In particular, the system shall enforce password expiry.

Previously used passwords shall not be allowed up to a certain number (Password History). 
The number of disallowed previously used passwords shall be:

-	Configurable;

-	Greater than 0;

-	And its default value shall be 3. This means that the network product shall store at least the three previously set passwords. The maximum number of passwords that the network product can store for each user is up to the manufacturer.

When a password is about to expire a password expiry notification shall be provided to the user.

Above requirements shall be applicable for all passwords used for the user account(e.g. application-level, OS-level, etc.). An exception to this requirement is machine accounts.

Pre-Conditions:

1.	Tester has account with username and password in the network product.

2.	Network product vendor will provide documentation for password management policy which should include details on how to change the password, configure password expiry rule and disallowing specified number of previously used passwords.  The default expiry time of the password and the default number of disallowed previously used passwords will also be provided.

3.	The network product vendor shall supply information on how many passwords the network product can store for each user in the password history.

4.	The tester has privilege to modify the number of disallowed previously used password.

Execution Steps

Execute the following steps:

A.	Positive Test

Case 1:

Test case to enforce password change after initial login is covered in clause 4.2.3.4.2.3.

Case 2:

1	The tester logs into network product application using a privileged account.

2	The network product application generates password expiry notification for user Y to force user Y to change the password.

3	The tester logs out as a privileged user and logs on as user Y.

4.	The tester is prompted to change his password and creates a new password by following the password policy management.

5	The network product application confirms change in password by, for example, displaying "Password Changed Successfully".

6	The tester successfully logs-in the network product application as user Y using the new password.

Case 3:

1.	The tester logs into network product application using a privileged account.

2.	Tester configures the network product application for number of disallowed previously used passwords to x

3.	The tester requests for a password change for user Y.

4.	The tester logs out of the privileged account and logs on as user Y

5. 	The tester creates a new password by following the password policy management.

6.	If the password is not equal to any of the x previously used passwords, the network product application still accepts the new password and displays "Password Changed Successfully".

Case 4:

1.  The tester logs into network product application using a privileged account.

2. the tester tests whether the Password History policy is enabled:

a). whether the password expiry time is enabled by default;

b). whether the number of disallowed previously used passwords is not set to 0.

B.	Negative Test

Case 1:

Test case to enforce password change after initial login is covered in clause 4.2.3.4.2.3.

Case 2:

No negative test case for this scenario.

Case 3:

1.	The tester logs into network product application using privileged account.

2.	Tester configures the network product application for number of disallowed previously used passwords to x for user Y.

3.	The tester logs out of the privileged account and logs in as user Y

4.	The tester requests for a password change.

5.	The tester sets the new password to a value that is among the last x passwords used previously x times.

Case 4：

No negative test case for this scenario.

Expected Results:

A.	Positive Test

Case 1:

Expected result for enforcing password change after initial login is covered in clause 4.2.3.4.2.3.

Case 2:

Tester can successfully change the password.

Case 3:

Tester can successfully change the password.

Case 4：

Tester confirm the password history policy is enabled by default.

B.	Negative Test

If the negative test case passes, this shows that network product application does not work properly and it violates the requirement.

Case 1:

Expected result for enforcing password change after initial login is covered in clause 4.2.3.4.2.3.

Case 2:

No negative test case for this scenario.

Case 3:

The tester cannot successfully change the password.

Case 4：

No negative test case for this scenario.

Expected format of evidence:

Evidence suitable for the interface, e.g. screenshot contains the operation result.

*************** End of Change ****************