3GPP TSG-SA3 Meeting #108-e 	S3-222191

e-meeting, 22 - 26 August 2022

Title:	[Draft] LS on security protection of RRCResumeRequest message

Response to:	N/A

Release:	Rel-18

Work Item:	SERP

Source:	SA3

To:	RAN, RAN2, RAN3

Cc:	N/A

Contact person:	Ivy Guo

Ivy_guo@Apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	NONE

# 1	Overall description

SA3 agreed on the solution#17 to protect RRCResumeRequest message in TR 33.809 for R18 in SA3#107-e, in the WID SERP, SA3 is working on the CR to complete the feature in R18.

In this solution#17, when the UE initiates the RRC Resume procedure, the UE should use the whole RRCResumeRequest message, except the ResumeMAC-I/shortResumeMAC-I, as an additional input parameter to the VarResumeMac-Input part in order to calculate ResumeMAC-I/shortResumeMAC-I. The UE should send the calculated ResumeMAC-I/shortResumeMAC-I in the RRCResumeRequest message. The UE and the network negotiate/learn each other's capability/support of using the newer version of ResumeMAC-I/shortResumeMAC-I as below:

-	UE's capability is part of an RRC message (i.e., AS SMComplete).

-	gNB/ng-eNB should indicate its capability of supporting the new version of ResumeMAC-I in  SI message (i.e., SIB1, refer to a closely related feature called useFullResumeID in SIB1).

- 	gNB/ng-eNB should also indicate its capability of supporting the new version of ResumeMAC-I in RRCRelease message with SuspendConfig, which is sent after AS SMC thus protected.

If UE receives both source and target gNB/ng-eNB’s capability of supporting new ResumeMAC-I, UE will include the new new ResumeMAC-I in the ResumeRequest.

The ResumeMac-I verification is done by the source gNB and not the target gNB. Target RAN node would potentially need to provide e.g the entire or partial RRC Resume Request Message to the last serving NG-RAN node in the XnAP RETRIEVE UE CONTEXT REQUEST message.

SA3 kindly request RAN2/3 to work on the corresponding specifications to support this feature, including the enhancement on the SIB message and RRCRelease message to indicate the gNB/ng-eNB’s support for the new resumeMAC-I calculation, and the enhancement of the XnAP RETRIEVE UE CONTEXT REQUEST message.

# 2	Actions

To RAN, RAN2, RAN3:

ACTION: 	SA3 kindly asks RAN/RAN2/RAN3 to take the above information into consideration and make changes in the corresponding specifications.

# 3	Dates of next TSG SA WG 3 meetings

SA3#108bis-e-Bis  	    10 -14 Oct 2022               	Electronic meeting

SA3#109  	                   14 -18 Nov 2022               	Canada