3GPP TSG-SA3 Meeting #108e 	S3-222201

# e-meeting, 22 - 26 August 2022										            		    Revision of S3-22xxxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.503 | CR | 0037 | rev | <Rev#> | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Key Definitions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some of the current key definitions cannot well refelct the association between the key and the service and the association between the key and the PC5 link, e.g. whether the key is used by a specific service or is generic which can be used by multiple services; whether the key is used by a specific PC5 link or is generic which can be used by multiple PC5 links. Actually, the association between the key and the service/PC5 link can be reflect by the input parameters to the key derivation function. Therefore, it is proposed to add more clarification for the current key definitions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extended/modified some of the key definitions with more clarification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear whether the root key(s) can be used for multiple services or only a specific service. It is not clear whether the root key(s) can be used for multiple PC5 links or only one specific PC5 link. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.2.3, 6.3.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the 1st Change ****************

6.3.3.2.3	PC5 Key Hierarchy over User Plane

Figure 6.3.3.2.3-1: PC5 Key Hierarchy for 5G ProSe UE-to-Network Relay security over User Plane

The different layers of keys (see figure 6.3.3.2.3-1) are the following:

-	PRUK: The root key for PC5 unicast link(s) for 5G ProSe UE-to-Network Relay services.

-	KNRP: The key derived from PRUK and is established for one PC5 unicast link between two UEs for a specific 5G ProSe UE-to-Network Relay service. This key is derived as specified in clause A.8.

-	KNRP-SESS: This intermediate key is derived as specified in TS 33.536 [6].

-	NRPEK, NRPIK: These session keys are derived as specified in TS 33.536 [6].

*************** Start of the 2nd Change ****************

6.3.3.3.3	PC5 Key Hierarchy over Control Plane

Figure 6.3.3.3.3-1: PC5 Key Hierarchy for 5G ProSe UE-to-Network Relay security over Control Plane

The different layers of keys (see figure 6.3.3.3.3-1) are the following:

-	KAUSF_P: A key derived based on 5G ProSe Remote UE specific authentication, only used to derive 5GPRUK.

-	5GPRUK: The root credential derived from KAUSF_P that is the root of security of the PC5 unicast link(s) used for a specific 5G ProSe UE-to-Network Relay service.

-	KNR_ProSe: This is a 256-bit root key that is established for one PC5 unicast link between two UEs for a specific 5G ProSe UE-to-Network Relay service.

-	Krelay-sess: This is the 256-bit intermediate key that is derived by the UE from KNR_ProSe and is used to derive keys to protect the transfer of data between the UEs. The Krelay-sess is derived per unicast link same as KNRP-sess specified in TS 33.536 [6]. During activated unicast communication session between the UEs, the Krelay-sess may be refreshed by running the rekeying procedure. The keys for confidentiality and integrity algorithms are derived directly from Krelay-sess. The 16-bit Krelay-sess ID identifies the Krelay-sess.

-	Krelay-int, Krelay-enc: The Krelay-int and Krelay-enc are session keys used in the chosen confidentiality and integrity algorithms respectively for protecting PC5-S signalling, PC5 RRC signalling, and PC5 user plane data. These keys are equivalent to NRPIK and NRPEK as specified in TS 33.536 [6]. They are derived from Krelay-sess and are refreshed automatically every time Krelay-sess is changed.

*************** End of the Changes ****************