3GPP TSG-SA3 Meeting #108e 	S3-221770_r1

- e-meeting, 22 - 26 August 2022

- Source:	Nokia, Nokia Shanghai Bell, Cablelabs

- Title:	Solution for UE privacy

- Document for:	Approval, Information, Discussion

Agenda Item:	5.13

# 1	Decision/action requested

It is requested to approve the new solution for UE privacy.

# 2	References

[x]	3GPP TR 33.887 Study on Security aspects for 5WWC Phase 2

[y]	3GPP TR 23700-17 "Study on the support for 5WWC, Phase 2"

# 3	Rationale

As per the KI#2 of 5WWC study defined in TR 33.887 [x], UE is able to discover the slice of TNGF/N3IWF and select TNGF/N3IWF accordingly. Please refer to the solution 13 and solution 14 of SA2 TR 3GPP TR 23700-17[y]

| Solution 13 : UE getting the slices supported by N3IWF (s) | The solution assumes that the UE(s) can themselves select the N3IWF that supports the set of slices they wish to use. For this, the UE sends a Slice Support Get request to each N3IWF. |
| --- | --- |
| Solution 14: use of AN NRF | The UE performs a N3IWF discovery procedure to a new “AN NRF” (similar to 5GC NRF defined in TS 23.501 [2]), to which N3IWFs register the set of Slices (S-NSSAI(s)) they support (reusing Nnrf_NFManagement_NFRegister service operation as defined in TS 23.502 [3] or a new similar service operation supported by a new NF) . |

In both of the above solutions, UE and network node provides slice information without any protection. Due to this, MitM will be able to check what slices/services UE is interested in. Therefore, it will leak the privacy information as agreed in the KI#2.

# 4	Detailed proposal

**** START OF CHANGE ****

## 6.Y	Solution #Y: Protection of privacy for UE Slice Information

### 6.Y.1	Introduction

This solution addresses the KI#2 of the 5WWC study defined in TR 33.887 [x]. If the UE and network node provide/share slice information (i.e. S-NSSAIs) directly without any protection, MitM will be able to determine which slices/services the UE is interested in. Therefore, it will leak the privacy information of the UE. This solution proposes the NSASG information sharing between the UE and network node (AN-NRF, N3IWF, TNGF) instead of S-NSSAI directly. By sharing the NSASG information, user privacy can be protected.

### 6.Y.2	Solution details

- 6.X.2.1	Procedure

- The list of mapping of S-NSSAI to Network Slice Access Stratum Groups (NSASGs) is configured at the 5G node (AN-NRF/N3IWF/TNGF). When UE sends a request to 5G node, it will include the NSASG information instead of S-NSSAI informaiton in the service request.

- 5G node shall map the NSASG information to S-NSSAI information and prepare the response to UE.

- 5G node shall provide node address.

### 6.Y.3	Evaluation

TBD

**** END OF CHANGE ****