3GPP TSG-SA3 Meeting #108e 	draft-S3-221931-r6

- e-meeting, 22 - 26 August 2022

- Source:	Ericsson, Lenovo, Nokia, Nokia Shanghai Bell, Rakuten Mobile Inc., Interdigital, US NSA, Motorola Solutions, Johns Hopkins University APL, Intel, Center for Internet Security, China Mobile, ZTE, CableLabs

- Title:	Key Issue#1 Need for trust evidence collection for continuous trust monitoring and evaluation

- Document for:	Approval

Agenda Item:	5.21

# 1	Decision/action requested

This pCR adds a key issue to TR 33.894.

# 2	References

[1]	SP-220674, ‘New Study on applicability of the Zero Trust Security principles in mobile networks’, Rel.18.

[2] NIST Special Publication 800-207, ‘Zero Trust Architecture’, August 2020.

# 3	Rationale

The tenets of Zero Trust include continuous monitoring of integrity and security posture of all owned and associated assets, network infrastructure, and communications [2, tenets 4, 5, 7]. Continuous trust monitoring is one of the key enablers for providing fine-grained access control decisions on a per-session basis to minimize impacts if any security incidents occur. To perform continuous trust monitoring, there is a need for data, that allows for observation of the security status of 5GC NFs and making decisions based on such monitoring. Currently, there is no standardised feature in 5GS that can provide trust evidence data that would supply continuous trust monitoring of 5GC NFs. Therefore, this pCR proposes a key issue on introducing key components to support continuous trust monitoring of 5GC NFs.

S3-221956 (KI#1 Need for continuous trust evaluation) are merged in S3-221931.

# 4	Detailed proposal

SA3 is kindly requested to agree on the pCR below to TR 33.894.

*****Start of Change 1*****

## 5.X	Key Issue #1: Need for trust evidence collection for continuous trust monitoring and evaluation/monitorn

### 5.X.1	Key issue details

The 5G system includes heterogenous and varied network functions (NF) deployments which may face issues such as the NF may run into errors due to configuration issues, the NF may get exposed to insider threats or cyber attacks etc., Once a NF is compromised or under attack, the services offered by the NF may also be impacted. Lateral movement of the threat to other connected NF can further increase the threat surface.

Currently, there is no standardised feature in 5GS that can automatically collect and provide continuous dynami monitoing and evaluation/monitoring wihsary data on. 5GC Nsrocedures for gathering and action of relevant intes that support apl, data encryption and te prot

The continuous trust monitoring and evaluationmg is need to apply appropriate security controls based on operator policies to detect and mitigate any anomalies.

### 5.X.2	Security threats

Based on ZT principles, assume that NF is constantly under threat or compromised in the core network, lack of data for continuous trust monitoring and evaluationmwill allow the impacted NF(s) to remain unidentified in the network and involved in various network services, which may lead to a service failure, data loss/theft, or lateral movement of the attack.

### 5.X.3	Potential security requirements

The 5GC shall support automatic and secure collection, transfer, and storing of the data for  continous trust monitoring and evaluation/monitoi

NOTE 1: The algorithms or logic for trust evaluation/monitroing and the outcome of trust monitoring and evaluationmare outside the scope of 3GPP.

NOTE 2: Which data that needs to be collected is part of this study.

*****End of Change 1*****