3GPP TSG-SA3 Meeting #108e 	draft_S3-222234-r2

- e-meeting, 22 - 26 August 2022

- Source:	Xiaomi

- Title: 	KI1, New Sol Authentication for devices that do not support 5GC NAS over WLAN access in NPN scenarios

- Document for:	Approval

Agenda Item:	5.16

# 1	Decision/action requested

It is requested to approve the new solution for KI #1 of 3GPP TR 33.858 v0.1.0.

# 2	References

[1]	3GPP TR 33.858: "Study on security aspects of enhanced support of Non-Public Networks (NPN) phase 2"

# 3	Rationale

This solution addresses secure non-3GPP access issue of key issue #1 in 3GPP TR 33.858 [1].

# 4	Detailed proposal

*************** Start of 1st Change ****************

## 6.A	Solution #A: < Authentication for devices that do not support 5GC NAS over WLAN access in SNPN scenarios >

### 6.A.1	Introduction

This solution addresses key issue #1.

TR 23.700-08 [2] studies "Key Issue #2: Support of Non-3GPP access for SNPN". Clause 5.2.1 of TR 23.700-08 [2] states: "Currently the 3GPP specifications do not support direct connection to SNPN via non-3GPP access networks" and "One objective of this key issue is to enable the 5GS to support direct connection of non-3GPP access networks to the SNPN's 5GC."

Devices that do not support 5GC NAS signalling over WLAN access (referred to as "Non-5G-Capable over WLAN" devices, or N5CW devices for short), may access 5GC in a SNPN via a trusted WLAN Access Network that supports a Trusted WLAN Interworking Function (TWIF).

To access to SNPN, N5CW device may need to provide the new identity (e.g., onboarding SUCI/onboarding SUPI) and the new Registration Type (i.e.  SNPN Onboarding) to non-3GPP WLAN Access Network.

This solution enables N5CW to access SNPN via trusted non-3GPP WLAN Access Network.

### 6.A.2	Solution details

The Trusted WLAN Interworking Function (TWIF) provides interworking functionality that enables connectivity with 5GC and implements the NAS protocol stack and exchanges NAS messages with the AMF on behalf of the N5CW device. Key-generating EAP-based authentication procedure is executed for connecting the N5CW device both to the trusted WLAN access network and to the 5G core network.

Figure 6.A.2-x: Authentication Procedure for N5CW

0.	The N5CW device selects a SNPN and a trusted WLAN that supports "5G connectivity-without-NAS“. "Access Network selection for devices that do not support 5GC NAS over WLAN".

Steps 1-10:	Initial registration to 5GC.

1.	The N5CW device associates with the trusted WLAN network and the key-generating EAP-based authentication procedure is initiated.

2.	N5CW sends UE identity (e.g., SUCI/on boarding SUCI/anonymous value SUCI) and AN parameters to the TWAP/TWIF. And SNPN identifier, which includes PLMN ID and NID, should be included in AN parameter. For SNPN Onboarding registration, the UE shall use Onboarding SUCI as specified in TS 33.501[X]. Moreover, in SNPN scenarios, if construction of SUCI as described in clause 6.12 of TS 33.501 cannot be used and if the EAP method supports SUPI privacy, then the UE may send an anonymous value SUCI based on configuration

If the UE is accessing 5GS for Onboarding, the AN parameters sent from UE to TWAP /TWIF shall include Onboarding indication. And the Registration Type should set as "SNPN Onboarding", which indicates that the UE wants to perform SNPN Onboarding Registration (i.e. allows the UE to access an ON-SNPN for the purpose of provisioning the UE with SO-SNPN credentials).

The remaining part of step 2 is identical to step 2 of clause 7A.2.4 of TS 33.501 [X].

3.	The TWIF shall create a 5GC Registration Request message on behalf of the N5CW device. The TWIF shall use UE identity, AN parameters, Registration Type that are sent by the UE in the Registration Request message.

4.	Step 4 is identical to step 4 of clause 7A.2.4 of TS 33.501 [X]..

5.

The Nausf_UEAuthentication_Authenticate Request message contains SUCI/onboarding SUCI/SUPI (in case of a valid 5G-GUTI is received by the AMF)/onboarding SUPI (in case of a valid 5G-GUTI is received by the AMF).

When the AMF receives a NAS Registration Request with the 5GS Registration Type set to "SNPN Onboarding", the AMF applies locally configured AMF Configuration Data for Onboarding in order to restrict UE network usage to only onboarding and stores in the UE Context in AMF an indication that the UE is registered for onboarding.

The remaining part of step 5 is identical to step 5 of clause 7A.2.4 of TS 33.501 [X].

6. As described in clause I.2 of TS 33.501 [X], the authentication mechanisms utilized in step 6 include key-generating EAP authentication methods.

The remaining part of step 6 is identical to step 8 of clause 7A.2.4 of TS 33.501 [X].

7.	It is identical to step 9 of clause 7A.2.4 of TS 33.501 [X]

8a.	It is identical to step 10a of clause 7A.2.4 of TS 33.501 [X]

8b. It is identical to step 10b of clause 7A.2.4 of TS 33.501 [X]

8c. It is identical to step 10c of clause 7A.2.4 of TS 33.501 [X]

9.	It is identical to step 11 of clause 7A.2.4 of TS 33.501 [X]

10.	It is identical to step 12 of clause 7A.2.4 of TS 33.501 [X]

11. It is identical to step 13 of clause 7A.2.4 of TS 33.501 [X]

Editor’s Note: Whether onboarding for N5CW devices is in scope is FFS.

### 6.A.3		System impact

TBD

### 6.A.4	Evaluation

TBD

*************** End of 1st Change ****************

*************** Start of 2nd Change ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-08: "Study on enhanced support of Non-Public Networks; Phase 2".

[3]	3GPP TS 22.261: "Service requirements for the 5G system".

…

[X]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

*************** End of 2nd Change ****************