# 3GPP TSG-SA3 Meeting #108-e	S3-221934

# e-meeting, 22nd – 26th August, 2022

# Source: 	Huawei, HiSilicon

# Title:	New authorization mechanism for the involved NFs in the delegated "Subscribe-Notify" scenarios

# Document for:	Approval

Agenda Item:	5.24

1	Decision/action requested

It is proposed to approve the change described in this document.

2	References

[1]	3GPP TR 33.875: " Study on enhanced security aspects of the 5G Service Based Architecture (SBA); ".

3	Rationale

It is proposed to add a new authorization mechanism for the involved NFs in the delegated “Subscribe-Notify” scenarios.

4	Detailed proposal

*** 1st CHANGE ***

6.X			Solution #X: Authorization mechanism for the involved NFs in the delegated “Subscribe-Notify” scenario.

6.X.1 Introduction

This solution addresses KI#3 on how to assure that the notification messages could be only forwarded to an authorized NF in the delegated “Subscribe-Notify” scenarios.

This solution proposes to include two instance IDs in the access token request, allowing the NRF to check whether one NF (e.g. NF_C) is authorized to subscription and whether another NF (e.g. NF_A) as the proxy is allowed to request the service from the identified NF Service Producer on behalf the NF_C. The NF Service Producer verifies the access token is valid according to these instance IDs. After successful verification, the NF Service Producer should provide the notification service.

6.X.2 Solution details

In the “Subscribe-Notify” NF service illustration 2 (delegated scenario) specified in TS 23.501 [3], clause 7.1.2, an NF Service Consumer (e.g. NF_C) may subscribe the service of an NF Service Producer (e.g. NF_B) on behalf of another NF Service Consumer (e.g. NF_A).

The Figure 6.X.2-1 describes the detailed solution for authorization mechanism for the involved NFs in the delegated “Subscribe-Notify” scenarios.

Figure 6.X.2-1: Authorization mechanism for the involved NFs in the delegated “Subscribe-Notify” scenario

- The NF_A decides to subscribe the service of NF Service Producer (e.g. NF_B) on behalf of NF_C and gets an access token from the NRF for a Service Request toward the NF_A. The NF_C initiates an NF service request to the NF_A which includes the notification URI, the access_token_NF_C and the CCA of NF_C to be used for subscription.

- The NF_A should verify if the access token and the CCA of the NF_C is valid and executes the service.

- The NF_A sends a Nnrf_AccessToken_Get request to NRF including the information to identify the target NF (NF Service Producer), the source NF (NF Service Consumer e.g. NF_C), the NF Instance ID of NF_A and the CCA_NF_C received at step 2.

- The NRF should check whether the NF_C are allowed to access the service provided by the identified NF Service Producer, and whether the NF_A as the proxy is allowed to request the service from the identified NF Service Producer on behalf the NF_C. The NRF authenticates NF_C based on the CCA of NF_C.

- If the authorization is successful, the NRF shall then generate an access token with the identity of the NF_A and the identity of the NF_C, NF type of the NF Service Producer (audience), subscribe service name(s), (scope).

The NRF sends access token to the NF_A in the Nnrf_AccessToken_Get response operation.

- The NF_A requests service to the NF Service Producer. The Service Request also consists of the CCA_NF_C, so that the NF Service Producer authenticates the NF _C.

- The NF Service Producer authenticates the NF_C and verifies the access token to ensure that the access token is valid. After authentication and authorization is successful, the NF Service Producer assures that the NF_A as the proxy is allowed to receive the response message on behalf the NF_C, and executes the subscribe service.

- The NF Service Producer should respond the NF_A and provide the NF_C with the notification service via NF_A.

6.X.3 Evaluation

TBD

*** END OF CHANGES***