3GPP TSG-SA3 Meeting #108e	S3-221803

# e-meeting, 22 - 26 August 2022

# Source:	Huawei, HiSilicon

# Title:	Solution on temporay slice authorization

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

Approve the new solution proposal to eNS3 TR33.886

# 2	References

[1]

# 3	Rationale

The contribution proposes a new solution to the KI#2.

# 4	Detailed proposal

pCR

***  BEGINNING OF CHANGES  ***

## 5.Y	Solution #Y: temporary slice authorization

### 5.Y.1	Introduction

This solution addresses the Key Issue #2: temporary slice authorization and slice service area authorization.

In order to support gracefully terminate a temporary slice and avoid abrupt PDU Session release, the UE authorized to access the temporary slice should be informed the duration of the temporary slice during UE registration.

### 5.Y.2	Solution details

The details of informing a UE the duration of the temporary slice is illustrated in the figure below.

1. UE sends AMF a Registration Request with a list of S-NSSAIs as specified in clause 16.2 of TS33.501.

2. AMF receives subscription information from UDM. For temporary slices, the termination time or lifetime of the slices are indicated.

3. AMF sends the Registration Accept message. For the authorized temporary slices with their S-NSSAIs included in the allowed NSSAI, the corresponding termination time or lifetime should also be included.

4. AMF initiates NSSAA procedures for those slices requiring NSSAA.

5. AAA-S send the NSSAA results to NSSAA which forwards to AMF. If successful, the message contains the authorized termination time or lifetime by AAA-S.

6. AMF updates the UE with the UE Configuration Update procedure, where allowed NSSAI is included. For temporay slices, AMF include the corresponding termination time or lifetime as well.

Figure 5.Y.2-1: temporary slice authorization

### 5.Y.3	Evaluation

TBC

***	END OF CHANGES	***