3GPP TSG-SA WG3 Meeting #108e	S3-221912

eMeeting, 22 - 26 August 2022

# Source:	Ericsson

# Title:	New solution for PC5 link security when UE-to-UE relay is in coverage

# Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This document proposes to add a new solution for UE-to-UE relay.

# 2	References

[1]	3GPP TR 23.700-33

[2]	3GPP TR 33.740

# 3	Rationale

This contribution proposes a new solution for UE-to-UE relay for KI#2 & KI#3 in TR 33.740.

# 4	Detailed proposal

**** START OF CHANGE ****

## 6.X	Solution #X: PC5 security establishment when UE-to-UE relay is in coverage

### 6.X.1	Introduction

This solution addresses Key issue #2: Security of UE-to-UE Relay and Key issue #3: Authorization in the UE-to-UE Relay Scenario.

For UE-to-UE relay use cases, the UE-to-UE relay may be in or out of 3GPP coverage. This solution provides a mechanism for PC5 security setup procedure between a source UE or target UE and a UE-to-UE relay when the UE-to-UE relay is in 3GPP coverage.

This solution assumes 5GC NFs e.g., 5GDDNMF and PKMF are deployed in the network.

### 6.X.2	Solution details

Figure 6.X.2-1 illustrates the high-level procedure of the proposed solution.

Figure 6.X.2-1: High-level procedure of PC5 security between Source/Target UE and UE-to-UE relay

0. 	The 5G ProSe Source/Target UE and UE-to-UE relay are provisioned with the discovery security materials and/or PRUK when they are in coverage.

Editor Note: Further provisioned parameters e.g., PC5 security policies of UE-to-UE relay are FFS.

1.	The discovery procedure for UE-to-UE Relay is performed by the 5G ProSe Source UE using the discovery parameters and discovery security material, based on the Relay Service Code for UE-to-UE Relay. If the UE-to-UE Relay is in 3GPP coverage, it also indicates whether network-based Relay service authentication and authorization is supported for UE-to-UE relay in the discovery announcement message.

2.	If the discovered UE-to-UE Relay supports network-based Relay service authentication and authorization, the 5G ProSe Source UE sends a Direct Communication Request (DCR) that contains PRUK ID or SUCI, Relay Service Code (RSC) of the 5G ProSe UE-to-UE Relay service and KNRP freshness parameter 1 to the 5G ProSe UE-to-UE Relay.

3.	The 5G ProSe UE-to-UE Relay sends a Key Request message that contains PRUK ID or SUCI, RSC and KNRP freshness parameter 1 to the 5GC.

Note: 5GC NFs and internal signalling are not described in detail.here for brevity. The similar security procedure as Security for 5G ProSe Communication via 5G ProSe Layer-3 UE to-Network Relay as defined in TS33.503 [6] can be reused.

4.	The 5GC sends the Key Response message to the 5G ProSe UE-to-UE Relay, which includes KNRP, KNRP freshness parameter 2.

Editor Note: Further input parameters in the Key Response message are FFS.

5a.	The 5G ProSe UE-to-UE Relay shall derive the session key (KNRP-SESS) from KNRP and then derive the confidentiality key (NRPEK) (if applicable) and integrity key (NRPIK) based on the PC5 security policies as specified in TS 33.536 [9]. The 5G ProSe UE-to-UE Relay sends a Direct Security Mode Command message to the 5G ProSe Source UE and include KNRP Freshness Parameter 2 in the message.

5b.	The 5G ProSe Source UE shall derive KNRP from its PRUK, RSC, KNRP Freshness Parameter 1 and the received KNRP Freshness Parameter 2 and then derive the session key (KNRP-SESS) and the confidentiality key (NRPEK) (if applicable) and integrity key (NRPIK) based on the PC5 security policies in the same manner as the 5G ProSe UE-to-UE Relay and process the Direct Security Mode Command. Successful verification of the Direct Security Mode Command assures the 5G ProSe Source UE that the 5G ProSe UE-to-UE Relay is authorized to provide the UE-to-UE relay service.

5c.	The 5G ProSe Source UE responds with a Direct Security Mode Complete message to the 5G ProSe UE-to-UE Relay.

5d.	On receiving the Direct Security Mode Complete message, the 5G ProSe UE-to-UE Relay shall verify the Direct Security Mode Complete message. Successful verification of the Direct Security Mode Complete message assures the 5G ProSe UE-to-UE Relay that the 5G ProSe Source UE is authorized to get the UE-to-UE relay service.

6.	The 5G ProSe Source UE and 5G ProSe UE-to-UE Relay continues the rest of procedure for the UE-to-UE relay service over the secure PC5 link.

7.	Step 1-6 are repeated for PC5 security establishment between the 5G ProSe Target UE and 5G ProSe UE-to-UE Relay.

8. 	The 5G ProSe Source UE and the 5G ProSe Target UE may establish an end-to-end Security via 5G ProSe UE-to-UE relay. The detail is not described in this solution.

### 6.X.3	Evaluation

**** END OF CHANGE ****