3GPP TSG-SA3 Meeting #108-e 	S3-221940

# e-meeting, 22nd – 26th August, 2022                                        		Revision of S3-22xxxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | draft-CR | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New test case to include SNPN snenario in token verification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph2 |  |  |  |  |  | Date: |  |  | 2022-08-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to TS 33.501, for SNPNs with Credentials Holder using AUSF and UDM for primary authentication, the NF Service Producer verifies the SNPN ID of the serving SNPN contained in the API request instead of the PLMN-ID, and the SNPN ID or the PLMN ID of the Credentials Holder instead of the home PLMN ID. It's proposed to add such scenario in token Vefirication TC to capture the above requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add SNPN scenario in token verification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case is missed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.2.3.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of 1st Change ****************.

4.2.2.2.3.x 	Authorization token verification failure handling in different SNPNs

Requirement Name: Authorization token verification failure handling in different SNPNs

Requirement Reference: TS 33.501 [10], clause 13.4.1.2

Requirement Description:

"The NF service producer shall check that the home PLMN ID of audience claim in the access token matches its own PLMN identity.

For SNPNs with Credentials Holder using AUSF and UDM for primary authentication, the NF Service Producer verifies the SNPN ID of the serving SNPN contained in the API request instead of the PLMN-ID, and the SNPN ID or the PLMN ID of the Credentials Holder instead of the home PLMN ID"

Threat References:  TR 33.926 [4], clause 6.3.3.1, Incorrect Verification of Access Tokens

NOTE:	The test case below only applies to the NFs which support identifying and understanding the producerPlmnId claim.

Test Case:

Test Name: TC_AUTHORIZATION_TOKEN_VERIFICATION_FAILURE_DIFF_SNPN

Purpose:

Verify that the NF service producer does not grant service access if the verification of authorization token from a NF service consumer in a different SNPN fails.

NOTE x: The test case only applies to UDMs, NRFs and AUSFs in Credentials Holders using AUSF and UDM for primary authentication.

Procedure and execution steps:

Pre-Conditions:

-	Test environment with a NF service consumer and two SEPPs (one cSEPP, one pSEPP).

-	The NF service consumer and SEPPs may be simulated.

-	The network product under test has already mutually authenticated with the NF service consumer in a different SNPN via the SEPPs.

-	The tester has the NRF’s private key or the shared key.

-	The network product under test is preconfigured with the NRF’s public key or the shared key.

-	The tester shall have access to the interfaces of the NF service consumer and the network product under test.

Execution Steps

The network product under test receives the access token sent from the NF service consumer, verifies the access token in accordance with the execution steps in 4.2.2.1.2.1, with the following additional test cases:

Test Case 1: incorrect SNPN ID of the NF service producer in the access token

1)	The test computes an access token correctly, except that the SNPN ID in the producerPlmnId claim of the access token is empty or different from the home SNPN ID of the network product under test, and then includes the access token in the NF Service Request sent from the NF service consumer to the network product under test through the SEPPs.

2)	The network product under test receives the access token sent from the NF service consumer through the SEPPs, verifies that the SNPN ID in the producerPlmnId claim of the access token is different from its own home SNPN identity.

Test Case 2: absent SNPN ID of the NF service producer in the access token

1)	The test computes an access token correctly, except that no producerPlmnId claim is included in the access token, and then includes the access token in the NF Service Request sent from the NF service consumer to the network product under test through the SEPPs.

2)	The network product under test receives the access token sent from the NF service consumer through the SEPPs, verifies that the access token is not a token to be used by the NF service consumer in a different SNPN, based on the absence of SNPN ID of the NF service producer in the access token.

Expected Results:

For both test cases 1 and 2, the network product under test rejects the NF service consumer’s service request based on Oauth 2.0 error response defined in RFC 6749 [12].

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

*************** End of 1st Change ****************