3GPP TSG-SA3 Meeting #108-e 	S3-221969

# e-meeting, 22 - 26 August 2022

# Source:	OPPO

# Title:	New solution: Authentication procedure of UE-to-UE relay

# Document for:	Approval

Agenda Item:	5.3

1	Decision/action requested

This pCR proposes to solve the Key Issue #2 in TR 33.740[1]

2	References

[1] 	3GPP TR 33.740

[2] 	3GPP TS 23.304

3	Rationale

This contribution addresses the security requirements for key issue #2 in TR 33.740[1].

4	Detailed proposal

SA3 is kindly requested to agree to the below pCR to TR 33.740 [1].

**** START OF CHANGE ****

## 6.X	Authentication procedure of UE-to-UE relay

### 6.X.1	Introduction

This solution addresses security requirement for establishment a secure connection between the source UE and the target UE in the UE-to-UE relay scenario in key issue #2.

After UE-to-UE Relay discovery and selection, source UE and target UE start to establish connection via the selected UE-to-UE relay. In the meanwhile, authentication between source UE and target UE, peer UEs and relay UE are performed to ensure the security of the connection establishment in this solution.

### 6.X.2	Solution details

### 6.X.2.1 Authentication procedure of UE-to-UE relay

Figure 6.X.1: Authentication procedure of UE-to-UE relay

- Pre-configuration: the source UE, UE-to-UE relay and target UE have been provisioned with the required credentials associated with their identity in advance, which contain a pair of public/private key. Each UE must act as "signer" and "verifier". As a signer, the UE uses its private key to sign an authentication message, and when acting as a verifier, the UE uses the signer’s public key to verify the signature.

Source UE has selected a suitable UE-to-UE Relay and received the Layer-2 ID of the target UE after Model A or Model B discovery. Source UE decides to connect with target UE via the selected UE-to-UE Relay.

- Source UE sends relay UE the Direct Communication Request message to trigger the link establihment and authentication between source UE and target UE via UE-to-UE relay, including Authentication_Info of source UE which contains the following parameters:

- User Info ID of Source UE

- Source UE’s public key

- SIGN-Source UE: A signature of the Authentication Request message

- Upon reception of the Direct Communication Request and Authentication Request message, UE-to-UE Relay verifies the signature SIGN-Source UE using Source UE’s public key. If the verification is successful, UE-to-UE Relay generates a signature of the Authentication Request message and the User Info ID of relay UE using its private key.

- Then UE-to-UE Relay sends the Direct Communication Request and Authentication Request message to the target UE, including Authentication_Info of source UE, Authentication_Info of relay UE (User Info ID of relay UE, relay UE’ public key, and SIGN1-relay).

- Upon reception of the Direct Communication Request and Authentication Request message, target UE verifies the signature SIGN-Source UE and SIGN1-relay. After successful verification, target UE generates a share key KD (this key only shared between source UE and target UE), using its private key and source UE’s public key.

- The target UE sends the Direct Communication Accept as described in clause 6.4.3.1 of TS 23.304 [2] to the UE-to-UE Relay, including the Authenticaiton Response which contains the following parameters:

- Authentication_Info of target UE: User Info ID of target UE, target UE’s public key, and SIGN-Target UE (i.e., the signature of the Authenticaiton Response message)

- Upon reception of the Direct Communication Accept and Authenticaiton Response message, UE-to-UE relay verifies the SIGN-Target UE using target UE’s public key. If the verification succeeds, UE-to-UE Relay generates a signature of the Authentication Response message and relay UE’Info using its private key.

- Then UE-to-UE Relay sends the Direct Communication Accept and Authenticaiton Response message to the source UE, including Authentication_Info of target UE, Authentication_Info of relay UE (User Info ID of relay UE, relay UE’ public key, and SIGN2-relay).

- The source UE verifies the signature SIGN-Target UE and SIGN2-relay. After successful verification, source UE generate a share key KD using its private key and target UE’s public key.

### X.X.3	Evaluation

TBD.

**** END OF CHANGE ****