3GPP TSG-SA3 Meeting #108-e 	S3-222060

# e-meeting, 22nd - 26th August 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1467 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on KIAB handling in Inter-donor migration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-07-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The new IP address of the migrating IAB-node DU will be taken into use during the migration for the F1 connections between the migrating IAB-node DU and source IAB-donor-CU. IAB-node DU IP address is used as input parameter in the KIAB derivation. The old KIAB derived with old IP address of the migrating IAB-node DU cannot be used for F1 connections during the migration as the IP address is already changed. The KgNB in IAB-node and Source IAB-donor are different, which results to the new psk (KIAB) mismatch between the migrating IAB-node DU and source IAB-donor-CU. The IKEv2 PSK authentication for the F1 connections during the migration will fail. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The source IAB-donor-CU will provide its own IP address to acquire the new KIAB. The target IAB-donor-CU will derive the new KIAB using new KgNB and IP address of the source IAB-donor-CU as well as IAB-node DU. After source IAB-donor-CU recived the new KIAB, the IKEv2 PSK authentication for the F1-C and F1-U connections will be performed with the new KIAB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IKEv2 PSK authentication for the F1 connections during the migration will fail. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex M.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

M.3.3.2	Security mechanisms for the F1 interface

The F1 interface connects the IAB-node (gNB-DU) to the IAB-donor-CU. It consists of the F1-C for control plane and the F1-U for the user plane.

F1 security for IAB is established using the security mechanisms for the F1 interface as specified in clause 9.8.2 of the present document, with IAB node taking the role of gNB-DU and IAB-donor-CU taking the role of gNB-CU.

In addition to the security mechanisms specified in clause 9.8.2 of the present document for the F1 interface, the IKEv2 Pre-shared Secret Key (PSK) authentication shall be supported. When IKEv2 performs a PSK authentication, in the IKE_AUTH request message, the IAB node shall set the ID type to ID_KEY-ID and set its value to PSK ID.

NOTE 1:	The PSK and PSK ID (for IKEv2 PSK authentication) can be preconfigured at the IAB node and IAB donor. Pre-configuration of the PSK(s) is out of the scope of the present document.

Additionally, to support a flexible plug and play of IAB-node and IAB-donor without a pre-configuration of the PSK(s), dynamic PSK computation for IKEv2 PSK authentication may also be supported. When dynamic PSK is used, the IAB-node and the IAB-donor shall calculate the PSK (KIAB) as specified in the Annex A.23 of this document. The IAB-donor shall uniquely identify the IAB-node’s security context (KgNB) using the IAB-node DU IP address. The IAB-donor shall use KIAB as PSK for IKEv2 between IAB-node and the IAB-donor. KIAB is stored in the IAB-node and in the IAB-donor. This key KIAB and the IPsec SA cryptographic keys are taken into use with the establishment of IPsec Security Association (SA) between the IAB-node and the IAB-donor. KIAB remains valid as long as the IAB-node is connected to the IAB-donor or until the IAB-node is re-authenticated.

In case of CP-UP separation of IAB-donor-CU (IAB-donor-CU contains IAB-donor-CU-CP and IAB-donor-CU-UP that use different IP address) then, IAB-donor-CU-CP and IAB-node DU shall generate KIAB-CU-CP and KIAB-CU-UP as specified in the Annex A.23 of this document. The key KIAB-CU-CP shall be used for establishment of secure F1 interface between the IAB-node DU and IAB-donor-CU-CP. The IAB-donor-CU-CP shall provide KIAB-CU-UP to the IAB-donor-CU-UP via E1 interface and KIAB-CU-UP shall be used for establishment of secure F1 interface between the IAB-node DU and IAB-donor-CU-UP.

In case of the inter-CU migration as specified in clause 8.17.3 and 8.17.4 in TS 38.401 [78], the source IAB-donor-CU shall provide its IP address to target IAB-donor-CU. The target IAB-donor-CU shall generate new KIAB for the source IAB-donor-CU with the source IAB-donor-CU IP address and new IAB-node DU IP address as specified in the Annex A.23 of this document. The target IAB-donor-CU shall provide new KIAB to the source IAB-donor-CU and the new KIAB shall be used for establishment of secure F1 interface between the IAB-node DU and source IAB-donor-CU.

NOTE 2:	KIAB is used as the PSK for IKEv2 authentication, the interface between the IAB-donor-CU and the SEG to provision the key KIAB in the SEG is implementation specific and out of the scope of the present document.

*** END CHANGES ***