3GPP TSG-SA3 Meeting #108e 	S3-222098

# e-meeting, 22 - 26 August 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.216 | CR | DRAFT CR | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | eNB checks UP IP capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph2 |  |  |  |  |  | Date: |  |  | 2022-08-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | eNB has supported UP IP, so the corresponding test case needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new requirement and test case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inaccurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********* Begin 1st change*********

4.2.2.1.x	Integrity protection of user data between the UE and the eNB

Requirement Name: Integrity protection of user data between the UE and the eNB.

Requirement Reference: TS 33.401 [3], clause 5.1.4.

Requirement Description: "User plane packets between the eNB and the UE may be integrity protected on the Uu interface. " in clause 5.1.4

Threat References: TBD

Test Case:

Test Name: TC-UP-DATA-INT_eNB

Purpose: To verify that the user data packets are integrity protected over the Uu interface.

Pre-Condition:

-	The eNB network product shall be connected in emulated/real network environments. UE may be simulated.

-	Tester shall have knowledge of integrity algorithm and integrity protection keys.

-	The tester can capture the message via the Uu interface, or can capture the message at the UE.

Execution Steps:

1. eNB sends RRCConnectionReconfiguration with integrity protection indication "on".

2. Check any User data sent by eNB after sending RRCConnectionReconfiguration and before UE enters CM-Idle state is integrity protected.

Expected Results:

Any user plane packets sent between UE and eNB over the Uu interface after eNB sending RRCConnectionReconfiguration is integrity protected.

Expected format of evidence:

Evidence suitable for the interface e.g. Screenshot containing the operational results.

********* End of change*********