3GPP TSG-SA3 Meeting #108-e 	S3-222193

# e-meeting, 22 - 26 August 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1474 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 33.501 to clarify the NAS COUNT usage in KeNB derivation in IRAT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-08-22 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In idle mode mobility from 5GS to EPS, it is only stated in 33501 that the KASME shall be derived from the KAMF using the 5G NAS Uplink COUNT value derived from the TAU Request message or Attach Request message in idle mode mobility, but what NAS COUNT should be used when deriving KeNB from KASME after idle mode mobility from 5GS to EPS is not clearly specified. In this case, a normal KeNB derivation should be followed, which is specified in TS 33.401 clause 7.2.7: “If there was no subsequent NAS SMC, the uplink NAS COUNT of the TAU request message sent from the UE to the MME is used as freshness parameter in the KeNB derivation ... If there is a NAS Security Mode Command after the TAU Request but before the AS SMC, the UE and MME use the NAS COUNT of the most recent NAS Security Mode Complete (i.e. the uplink NAS COUNT) and the related KASME as the parameter in the derivation of the KeNB. ” Therefore this CR clarified this case in TS 33.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description in 8.6.1 to clarify the NAS COUNT usage in deriving KeNB in idle mode mobility from N1 to S1 mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear description |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

++++++++++++++++++++++++++++++++ Begin Changes ++++++++++++++++++++++++++++++

## 8.6	Mapping of security contexts

### 8.6.1	Mapping of a 5G security context to an EPS security context

The derivation of a mapped EPS security context from a 5G security context is done as described below:

-	The KASME' key, taken as the KASME, shall be derived from the KAMF using the 5G NAS Uplink COUNT value derived from the TAU Request message or Attach Request message in idle mode mobility or the 5G NAS Downlink COUNT value in handovers as described in Annex A.14.

-	The eKSI for the newly derived KASME key shall be defined such as the value field is taken from the ngKSI and the type field is set to indicate a mapped security context.

-	The EPS uplink and downlink NAS COUNT values in the mapped context shall be set to the uplink and downlink NAS COUNT values of the current 5G security context respectively.

-	The selected EPS NAS algorithms shall be set to the EPS algorithms signalled to the UE by the AMF during an early authentication procedure followed by a NAS SMC as described in clause 6.7.2.

NOTE 1:	Whenever an algorithm change is required, the target MME initiates an NAS SMC to select other algorithms as described in TS 33.401 [10].

NOTE 2:	When deriving KeNB after idle mode mobility from 5G to 4G, the key derivation rules used for UE transition from idle to connected mode in TS 33.401 should be followed.

++++++++++++++++++++++++++ End of Changes ++++++++++++++++++++++++++++++