3GPP TSG-SA3 Meeting #108e 	S3-222235

# e-meeting, 22 – 26 Aug 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1480 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving the alignment related EN for NSACF Subscription/unsubscription procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is a EN "the procedure shall be aligned with SA2" in 16.6.3 of TS 33.501. In clause 16.6.3 of TS 33.501, ENSI is utilized to identify S-NSSAI for NEF and AF that is outside the 3GPP operator domain. According to clause 6.2.5.0 of TS 23.501, it is stated that "It(NEF) translates between information exchanged with the AF and information exchanged with the internal network function. For example, it (NEF) translates between an AF-Service-Identifier and internal 5G Core information such as DNN, S-NSSAI, as described in clause 5.6.7. " According to clause 4.15.3.2.10 of TS 23.502, SA2 leverages the AF-Service-Identifier to identify S-NSSAI for NEF and AF that is outside the 3GPP operator domain. Therefore, SA3 needs to clarify the relationship between ENSI and AF-Service-Identifier. |  |  |  |  |  |  |  |  |
| Summary of change: |  | EN "the procedure shall be aligned with SA2" in 16.6.3 of TS 33.501 is removed. Added the text clarifiying that the AF-Service-Identifier can be used as the ENSI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Terminology misalignment between SA2 and SA3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the Change ****************

16.6.3	Subscription/unsubscription procedure of NSACF notification service

Figure 16.6.3-1: Subscription/unsubscription of NSACF notification procedure

0.	Authentication of AF: AF is authenticated by NRF or authenticated by NEF based on description in clause 13 or clause 12. A token is generated for AF after authentication and authorization.

Editor’s Note: It is FFS how AF outside the 3GPP operator domain is authorized.

1.	To subscribe or unsubscribe for the number of UEs or the number of PDU Sessions per network slice notification with the NSACF, the AF sends Nnef_EventExposure_Subscribe/Unsubscribe Request (Event ID, Event Filter, Event Reporting information) message to the NEF as described in TS 23.502 [8]. The Event Filter parameter shall be ENSI for an AF deployed outside the 3GPP operator domain. Other parameters are specified in TS 23.502 [8]. AF-Service-Identifier defined in TS 23.502 [8] can be used as ENSI.

2.	The NEF confirms with Nnef_ SliceStatusEventExposure _Subscribe/Unsubscribe Response message to the AF.

The Event Filter parameter is the mapped ENSI for the AF deployed outside the 3GPP operator domain.

3.	The NEF checks whether the AF is authorised for the requested subscription based on the AF token. It needs to check whether the token claims match the AF’s identity and the Event Filter parameter. If authorised, the NEF may query the NRF to find the NSACF responsible for the requested S-NSSAI (NEF needs to map to S-NSSAI based on ENSI for the AF deployed outside the 3GPP operator domain).

4.  The NEF forwards the request to the NSACF with Nnsacf_SliceEventExposure_Subscribe/Unsubscribe Request (Event ID, Event Filter, Event Reporting information). The Event Filter parameter shall be the mapped S-NSSAI for the AF deployed outside the 3GPP operator domain.

5.	The NSACF confirms with Nnsacf_SliceEventExposure_Subscribe/Usubscribe Response message to the NEF as in TS 23.502 [8].

6-7a.	The NSACF triggers a notification towards the AF and sends the Nnsacf_SliceEvent Exposure_Notify (Event ID, Event Filter, Event Reporting information) message to the NEF as described in TS 23.502 [8].

7b-9.	The NEF forwards the message to the AF for single NSACF or aggregates reporting information for multiple NSACFs in the Nnef_EventExposure_Notify (Event ID, Event Filter, Event Reporting information) message as described in TS 23.502 [8]. The Event Filter parameter shall be the mapped ENSI from the S-NSSAI for the AF deployed outside the 3GPP operator domain.

*************** End of the Change ****************