| 3GPP TR 33.891 V0.2.0 (2022-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Security of Phase 2 for UAS, UAV and UAM (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Overview	7

5	Key Issues	7

5.1	Key issue #1: Direct C2 Security	7

5.1.1	Key issue details	7

5.1.2	Security threats	7

5.1.3	Potential security requirements	8

5.2	Key issue #2: Security of DAA unicast connection	8

5.2.1	Key issue details	8

5.2.2	Security threats	8

5.2.3	Potential security requirements	8

5.3	Key issue #3: Direct C2 Authorization	8

5.3.1	Key issue details	8

5.3.2	Security threats	8

5.3.3	Potential security requirement	9

5.4	Key issue #4: UAV/UAV-C Privacy over PC5 link for C2	9

5.4.1	Key issue details	9

5.4.2	Security threats	9

5.4.3	Potential security requirement	9

5.5	Key issue #5: Privacy protection over PC5 for DAA unicast messages	9

5.5.1	Key issue details	9

5.5.2	Security threats	10

5.5.3	Potential security requirements	10

6	Solutions	10

6.1	Solution #1: Security establishment and link security protection of unicast PC5 communication	10

6.1.1	Introduction	10

6.1.2	Solution details	10

6.1.3	Evaluation	11

6.2	Solution #2:  Solution to secure direct C2 and DAA connection	11

6.2.1	Introduction	11

6.2.2	Solution details	11

6.2.3	Evaluation	14

6.3	Solution #3: C2 and DAA unicast security using V2X unicast solution	14

6.3.1	Introduction	14

6.3.2	Solution details	14

6.3.3	Evaluation	15

6.4	Solution #4: Direct C2 communication over PC5 security	15

6.4.1	Introduction	15

6.4.2	Solution details	15

6.4.3	Evaluation	16

7	Conclusions	16

Annex <A>: <Informative annex title for a Technical Report>	17

Annex <X> (informative): Change history	18

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document investigates and identifies the security and privacy threats and corresponding security requirements for Uncrewed Aerial Vehicles (UAVs) and Urban Air Mobility (UAM) that derive from the architecture and system level enhancements studied in TR 23.700-58 [2]. Furthermore, the present document considers solutions and analyses these to make recommendations for possible normative work taking into consideration the conclusions of TR 23.700-58 [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-58: " Study of further architecture enhancements for uncrewed aerial systems and urban air mobility".

[3]	3GPP TS 22.125: "Uncrewed Aerial System (UAS) support in 3GPP".

[4]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking".

[5]	3GPP TS 33.256: "Security aspects of Uncrewed Aerial Systems (UAS)".

[6]	3GPP TS 33.536: "Security aspects of 3GPP support for advanced Vehicle-to-Everything (V2X) services".

[7]	3GPP TS 33.503: "Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[8]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

C2	Command and Control

DAA	Detect And Avoid

UAS	Uncrewed Aerial System

UAM	Urban Air Mobility

UAV	Uncrewed Aerial Vehicle

# 4	Overview

Some requirements for UAS (Uncrewed Aerial System) defined in TS 22.125 [3] are not addressed in TS 23.256 [4] and TS 33.256 [5], which includes:

- Direct C2 communication: the UAV controller and UAV establish a direct C2 link to communicate with each other and both are registered to the 5G network using the radio resource configured and scheduled provided by the 5G network.

- Broadcast UAS Remote Identification (Remote ID) services. UAS Remote ID refers to a UAS in flight provides identification and tracking information that can be received by regulatory agencies.

- Collision avoidance: UAV can navigate safely from one place to its goal without colliding with other UAVs or obstacles.

The requirements above are studied in TS 23.700-58 [2] for architecture aspects and this present document for security aspects, which covers:

-	a mechanism for Command and Control (C2) communications over PC5 interface in3GPP system;

-	a mechanism to transport Broadcast Remote Identification; and

-	a mechanism to support aviation applications such as Detect And Avoid (DAA).

# 5	Key Issues

## 5.1	Key issue #1: Direct C2 Security

### 5.1.1	Key issue details

In TR 23.700-58 [2], key issue #1 focuses on the transport of C2 communications over PC5 in the 3GPP system, while considering the following aspects:

"- how is the C2 communication over PC5 between a UAV and UAV controller established;

-	how is the UAV authorized for setting up direct C2 communication over PC5 with a UAV controller, both for in-coverage and out of coverage scenarios, and how is the authorization revoked;

-	whether the UAV needs to discover the UAV controller, or vice versa and if so, how?"

### 5.1.2	Security threats

The lack of security for the PC5 unicast link between UAV and UAV-C used for C2 communication may let the attackers to eavesdrop and control the UAV operations thereby leading to UAV hijack and misoperations.

### 5.1.3	Potential security requirements

The 3GPP system shall provide means for UAV and UAV-C to establish secure PC5 link used for C2 communication.

## 5.2	Key issue #2: Security of DAA unicast connection

### 5.2.1	Key issue details

Some solutions for transport of DAA traffic discuss using a unicast connection . In such cases, signalling might be needed to establish the connection between the appropriate entities and hence would need protection which could also be applied to the user plane.

### 5.2.2	Security threats

Signalling messages and data sent using a unicast connection for DAA can be modified or eavesdropped by an attacker.

### 5.2.3	Potential security requirements

The 5GS shall support the ability to confidentiality, integrity and replay protect any 3GPP signalling traffic used to establish and manage the unicast connection for DAA and any user plane of such a connection.

## 5.3	Key issue #3: Direct C2 Authorization

### 5.3.1	Key issue details

TS 23.256 [4] and TS 33.256 [5] describe how the UAV pairing with a UAV-C is authorized by a USS over a PDU Session for C2 communications performed over Uu.

In TR 23.700-58 [2], key issue #1 focuses on the support for transport of C2 communications over PC5 in the 3GPP system and considering the following aspects:

"- how is the C2 communication over PC5 between a UAV and UAV controller established;

-	how is the UAV authorized for setting up direct C2 communication over PC5 with a UAV controller, both for in-coverage and out of coverage scenarios, and how is the authorization revoked;

-	whether the UAV needs to discover the UAV controller, or vice versa and if so, how?"

The following service requirement in TS 22.125 [3] applies to the C2/pairing authorization, independently of the communication mode used to operate the UAV i.e., over Uu or direct over PC5:

[R-5.1-010] The 3GPP system shall enable UTM to inform an MNO of the outcome of an authorisation to operate.

This is further captured for direct C2 in TR 23.700-58 [2] architectural assumption:

-	A UAV shall obtain authorization from 3GPP system and USS to perform direct C2 communication as defined in TS 22.125 [4].

The solutions in TR 23.700-58 [2] (e.g., solution#1, #4) propose to support Direct C2 communication between UAV and UAV-C considering that UAV and UAV-C may be pre-paired or dynamically paired.

### 5.3.2	Security threats

If pairing authorization of UAV and UAV-C is not performed securely before establishment of a direct connection between the UAV and UAV-C, an unauthorized UAV-C may be able to communicate with the UAV and perform an unauthorized flight which could cause tremendous risks to the security of UAS and public safety.

If 3GPP system is not capable to handle revocation of the direct connectivity pairing authorization from USS, then USS might not be able to take appropriate measures to deal with misbehaving UAVs which might cause accidents or become attack vectors.

### 5.3.3	Potential security requirement

3GPP system shall support enabling authentication and authorization by the USS of a UAV and UAV-C pairing before enabling a direct data connection over PC5 between the UAV and UAV-C

3GPP system shall provide means for the USS to revoke a UAV and UAV-C pairing authorization in order to close the direct connection over PC5 between the UAV and UAV-C.

## 5.4	Key issue #4: UAV/UAV-C Privacy over PC5 link for C2

### 5.4.1	Key issue details

In TR 23.700-58 [2], key issue #1 focuses on the support for transport of C2 communications over PC5 in the 3GPP system and considering the following aspects:

"- how is the C2 communication over PC5 between a UAV and UAV controller established;

-	how is the UAV authorized for setting up direct C2 communication over PC5 with a UAV controller, both for in-coverage and out of coverage scenarios, and how is the authorization revoked;

-	whether the UAV needs to discover the UAV controller, or vice versa and if so, how?"

3GPP system has to be able to protect the privacy of the UE (UAV or UAV-C) privacy sensitive identities when using PC5 link during direct discovery or for direct communications over the PC5 unicast link.

NOTE 1: Broadcast Remote ID (BRID) is not considered in this key issue.

### 5.4.2	Security threats

### If privacy sensitive identities of the UE (i.e., UAV or UAV-C) are not protected during direct discovery or direct C2 communication over the PC5 unicast link procedures used for C2, an eavesdropper can perform privacy attacks on the UE (UAV/UAV-C).

### 5.4.3	Potential security requirement

The 5G System should provide means for mitigating trackability and linkability attacks on UAV/UAV-C during communications over the PC5 unicast link used for C2.

The 5G System should provide means for mitigating trackability and linkability attacks on UAV/UAV-C during discovery over the PC5 link used for C2.

## 5.5	Key issue #5: Privacy protection over PC5 for DAA unicast messages

### 5.5.1	Key issue details

KI#3 in TR 23.700-58 [2] addresses the support of DAA, and solutions such as #5 and#7 propose the unicast between entities over PC5 for DAA, this may introduce privacy issues for unicast messages related to long duration unicast session, e.g. source L2 ID tracking. If the same L2 ID is used by a UE for a certain period of time, it is possible that other UEs and/or attackers can track and identify the source of the message based on the PC5 parameters sent as part of the PC5 transmissions.

### 5.5.2	Security threats

An adversary that is capable of connecting and linking L2 identities to a real or a long-term Application Layer ID will be able to track and trace the endpoint in space and time. Such trackability and linkability will be an attack on U2X endpoint privacy.

### 5.5.3	Potential security requirements

The 5G System shall provide means for mitigating trackability attacks on L2 identities during U2X unicast communications over PC5 for DAA.

The 5G System shall provide means for mitigating linkability attacks on L2 identities during U2X unicast communications over PC5 for DAA.

# 6	Solutions

## 6.1	Solution #1: Security establishment and link security protection of unicast PC5 communication

### 6.1.1	Introduction

This solution addresses the direct PC5 communication (unicast) for both the direct C2 and unicast DAA scenarios as specified in Key Issue #1 and Key Issue #2, respectively. Generally, unicast security establishment specified in eV2X TS 33.536 [6] and 5G ProSe TS 33.503 [7] is reused as the baseline.

### 6.1.2	Solution details

The unicast PCF communication establishment starts with a Direct Communication Request (DCR) message to send the initiating UE’s security capabilities and to trigger the mutual authentication and key establishment. After mutual authentication and key establishment, the Direct Security Mode Command and the Direct Security Mode Complete messages are emitted to inform the selected security protection algorithms for the connection and the initiating UE’s user plane security policies (i.e. user plane confidentiality and integrity protection policies), respectively. Finally, the receiving UE replies a Direct Communication Accept (DCA) message to confirm the user plane protection methods and finish the unicast PC5 communication establishment procedures.

The signalling and user plane security protection are protected based on the detailed negotiation procedures:

0. UAS security-related parameter (for unicast secure communication over PC5) pre-configuration and previsioning, the signalling messages are integrity protected and the signalling ciphering protection is a configuration option.

NOTE 1:  Step 0 is done only in coverage.

1. UAS discovery procedures may happen for the UAV and UAVC to find each other in direct C2 scenario.

NOTE 2:  whether or not the UAV needs to discover the UAVC or vice versa will align with the decision in SA2.

2. The initiating UE (UAV or UAVC) starts DCR message contains its security capabilities and signalling security policy. The security capabilities are the confidentiality and integrity protection algorithms that the initiating UE accepts for this connection.

3. To generate the PC5 root key to protect the PC5 connection, the receiving UE may initiate the Direct authentication and key establishment procedures with the initiating UE.

4. The receiving UE uses the Chosen_algs to indicate the selected confidentiality and integrity protection algorithms of this link and contains the Chosen_algs in the Direct Security Mode Command message. The initiating UE’s security capabilities are sent back to the initiating UE to mitigate the bidding down attack. The receiving UE integrity protects the Direct Security Mode Command message before sending it to the initiating UE.

5. The initiating UE sends its user plane security policy to the receiving UE by using Direct Security Mode Complete message.

6. The receiving replies the DCA message to accept the DCR message and the unicast PC5 communication establishment including the user plane security indication. The user plane security protection methods (the user plane with or without confidentiality and/or integrity protection) are explicitly indicated by using the user plane security indication.

### 6.1.3	Evaluation

FFS

## 6.2	Solution #2:  Solution to secure direct C2 and DAA connection

### 6.2.1	Introduction

The solution addresses KI#1 and KI#2.

The solution provides an overview of direct U2X connection security which describes (i) how 5GS can provide means for UAV and UAV-C to establish secure PC5 link for Command and Control (C2) communication service(s) and (ii) how 5GS can support the ability to confidentiality, integrity and replay protect any 3GPP signalling traffic used to establish and manage the unicast connection for Detect and Avoid (DAA) service(s).

### 6.2.2	Solution details

UAV-to-Everything (U2X) services such as C2 and direct DAA can utilize PC5 link for establishing C2 connection (i.e., between UAV and UAV-C) and for establishing unicast connection for DAA (between UAVs) respectively as discussed in TR 23.700-58 [2].

The procedure to establish a secure U2X service direct communication is shown in the following figure 6.2.2-1.

Figure 6.2.2-1 Secure U2X service direct communication establishment

The steps shown in Figure 6.2.2-1 is described as follows:

1a-b.	If the UAV is capable of Uu communication, the UAV performs UUAA procedure and C2 authorization (if required) as described in TS 23.256 [4] and TS 33.256 [5]. The UAV obtains UAV-C pairing information (if not configured already) and U2X security policy for each U2X services (i.e., C2 and DAA) along with the result of successful C2 authorization. The U2X security policy includes signalling and user plane security policy per U2X service type (i.e., for C2 and DAA, the signalling and user plane confidentiality and integrity are set as required based on local policy), pairing restrictions list, and access restriction information (i.e., UAV information which are restricted to perform DAA).

Similarly, the UAV-C can also perform UUAA procedures and C2 authorization as described in TS 23.256 [4] and TS 33.256 [5]. The UAV-C obtains UAV pairing information (if not configured already), U2X security policy for each U2X services (i.e., C2 and DAA) along with the result of successful C2 authorization. Alternatively, thse data may be configured by the USS in UAV-C which is upto USS.

NOTE 1: UUAA and C2 authorization for UAV-C is not covered in Rel-17.

2.	The UAV if determines to set up C2 communication over PC5, it sends Direct Communication Request with U2X service type which indicates C2 service, UAV identifier (i.e., CAA-Level UAV ID), UAV-C identifier, U2X service security policy specific to the C2 service (i.e., confidentiality and integrity protection requirements for signalling and user plane protection), security capability and key establishment information (as described in TS 33.536 [6] ) which can be related to security information for C2 security.

3.	The UAV-C on receiving the direct communication request, if the U2X service type indicates C2 service, the UAV-C verifies the received U2X service security policy and UAV ID against the locally configured U2X security policy which includes the Pairing restrictions list and U2X service security policy. If the received UAV ID is same as any of the UAV ID(s) in the (authorized) pairing restrictions list and if the U2X service security policy matches with the locally stored one, then the UAV-C determines to respond with step 4.

If the UAV ID in the direct communication request do not match with any of the UAV ID(s) in the pairing restrictions list or if the received U2X service security policy violates the locally configured U2X service security policy for the C2 service, then the UAV-C determines to reject the direct communication, where the UAV-C skip steps 4-6a and performs step 6b.

4.  UAV-C performs direct authentication and key establishment (as described in TS 33.536 [6]) with the UAV.

5a. The UAV-C sends to the UAV, the Direct security mode command which includes (information as in TS 33.536 [6] ) Key_Est_Info, MSB of Key ID (e.g., KNRP ID to indicate the C2 security key), security capability, and additional information such as those received in step 1 (i.e., U2X service type, U2X service security policy). The session key (i.e., for C2), PC5 signalling and user plane keys (for confidentiality and the integrity) as required can be derived to protect the C2 service based on the U2X service security policy.

5b.	The UAV checks that the returned security capabilities, U2X service type and U2X service security policy are the same as those it sent in step 1. The UAV on receiving the Direct security mode command, if the above check is successful, based on received Key_Est_Info (as in TS 33.536 [6]) derives the key and choose LSB of Key ID (e.g., KNRP ID) to uniquely identify the Key and locally store the key with the identifier. Then the UAV sends to the UAV-C, the Direct security mode complete message which includes LSB of Key ID, security capabilities, U2X service type, and U2X service security policy sent in step 1. The confidentiality key and the integrity key as required can be derived to protect the C2 service based on the U2X service security policy.

6a.	The UAV-C sends Direct Communication Accept over the established link. The UAV and UAV-C can start C2 communication over PC5.

6b. The UAV-C based sends a direct communication reject message (i.e., if the U2X security policy is not met, or if authentication and key establishment fails or if the direct security mode command procedure fails) with respective cause information.

Security of DAA unicast connection:

Direct UAV to UAV communication for the purpose of DAA can use PC5 (e.g., C-V2X) as described in TR 23.700-58 [2] Clause 5.3. To enable confidentiality, integrity and relay protection for DAA related unicast connection, the procedure described using Figure 6.2.2-1 can be used with the following DAA specific adaptations.

2. If the UAV (e.g., UAV 1) determines to set up DAA connection over PC5, it sends Direct Communication Request with U2X service type which indicates DAA service, UAV identifier (i.e., CAA-Level UAV ID), U2X service security policy specific to the DAA service (i.e., confidentiality and integrity protection requirements for signalling and user plane protection), security capability and key establishment information (as described in TS 33.536) which can be related to security information for DAA security.

3.	The UAV (e.g., UAV 2) on receiving the direct communication request, if the U2X service type indicates DAA service, the UAV-C verifies the received U2X service security policy and UAV ID against the locally configured U2X security policy which includes the access restriction information (i.e., UAV information which are restricted to perform DAA) and U2X service security policy. If the received UAV ID not part of access restriction information, and if the U2X service security policy matches with the locally stored one, then the UAV determines to respond with step 4.

If the UAV ID in the direct communication request match with any of the UAV ID(s) in the access restriction information or if the received U2X service security policy violates the locally configured U2X service security policy for the DAA service, then the UAV-C determines to reject the direct communication, where the UAV skip steps 4-6a and performs step 6b.

4.  UAV (e.g., UAV 2) performs direct authentication and key establishment (as described in TS 33.536 [6]) with the UAV (e.g., UAV 1).

5a. The UAV (e.g., UAV 2) sends to the UAV (e.g., UAV 1), the Direct security mode command which includes (information as in TS 33.536 [6]) Key_Est_Info, MSB of Key ID (e.g., KNRP ID to indicate the DAA security key), security capability, and additional information such as those received in step 1 (i.e., U2X service type, U2X service security policy). The session key (i.e., for DAA), PC5 signalling and user plane keys (for confidentiality and the integrity) as required can be derived to protect the DAA service based on the U2X service security policy.

5b.	The UAV (e.g., UAV 1) checks that the returned security capabilities, U2X service type and U2X service security policy are the same as those it sent in step 1. The UAV on receiving the Direct security mode command, if the above check is successful, based on received Key_Est_Info (as in TS 33.536 [6]) derives the key and choose LSB of Key ID (e.g., KNRP ID) to uniquely identify the Key and locally store the key with the identifier. Then the UAV (e.g., UAV 1) sends to the UAV (e.g., UAV 2), the Direct security mode complete message which includes LSB of Key ID, security capabilities, U2X service type, and U2X service security policy sent in step 1. The confidentiality key and the integrity key as required can be derived to protect the DAA service based on the U2X service security policy.

6a.	The UAV (e.g., UAV 2) sends Direct Communication Accept over the established link. The UAVs can start DAA communication over PC5.

6b. The UAV (e.g., UAV 2) based sends a direct communication reject message (i.e., if the U2X security policy is not met, or if authentication and key establishment fails or if the direct security mode command procedure fails) with respective cause information.

### 6.2.3	Evaluation

TBD

## 6.3	Solution #3: C2 and DAA unicast security using V2X unicast solution

### 6.3.1	Introduction

This solution addresses both key issue #1 and #2. It re-uses the security procedures for unicast security for V2X (see TS 33.536 [6]).

### 6.3.2	Solution details

TS 33.536 [6] contains a method of establishing security for a unicast connection for V2X services. This solution proposes to re-use those procedures for establishing the security of C2 and DAA unicast connections. The relevant procedures are given in clause 5.3.3 of TS 33.356 [6].

Editor’s note: Whether there is a need for discovery before communication will be base on an SA2 decision.

Editor’s note: More details on the credentials for establishing DAA connections are FFS.

Editor’s note: Details on authorisation for C2 pairing are FFS.

Figure 6.3.2-1: Message flow for the establishment of unicast security

Figure 6.3.2-1 gives an overview of the flow for (re-)establishing security for unicast connections (see TS 33.536 [6] for the details). The security policy relevant to the service (i.e. DAA or C2) will be used when establishing a connection for that service.  C2 and DAA traffic will only be accepted from a peer authorised to send such traffic.

Editor’s note: The need for and security issues of DAA and C2 traffic sharing a connection are FFS.

### 6.3.3	Evaluation

The evaluation is FFS.

## 6.4	Solution #4: Direct C2 communication over PC5 security

### 6.4.1	Introduction

This solution addresses Key issue #1: Direct C2 Security.

In this solution, a UAV and a UAV-C establish a secure PC5 unicast link for C2 communication based on procedure described in TS 33.536 [6]. Both UAV and UAV-C supports PC5, with optional support for Uu connection.

The UAV that uses Uu connection is authenticated and authorized by the USS for C2 communication prior to establishing C2 communication over PC5, as per existing Rel-17 procedures. The UAV may receive security information for C2 over PC5 (e.g., key material) through these procedures, in the case of dynamic pairing with UAV-C. The UAV/UAV-c may also be preconfigured with security credentials (e.g., long term key) in the case of static pairing (i.e., pairing is pre-determined).

### 6.4.2	Solution details

Figure 6.4.2-1: PC5 unicast security establishment for C2 communication between UAV and UAV-C

1. 	The UAV performs UUAA procedure as described in TS 33.256 [5], clause 5.2.1. The UAV may obtain a new CAA-Level UAV ID through this procedure. The UAV uses the new CAA-Level UAV ID or a pre-configured CAA-Level UAV ID in the following steps.

2.	The UAV performs Pairing Authorization procedure as described TS 33.256 [5], clause 5.4. The UAV may obtain UAV-C identifier and security information (e.g., key material) during this procedure. The UAV uses the received information or pre-configured information during the following link establishment procedure.

3.	The UAV sends Direct Communication Request to initiate the PC5 unicast link establishment. The DCR includes:

-	Source User Info: the UAV's Application Layer ID (e.g., CAA-Level UAV ID or any other application layer ID assigned for C2 over PC5).

-	Target User Info: if the UAV-C identifier is known (e.g., received in step 2 or pre-configured), the UAV uses it as the target user info. If the UAV-C identifier is not available, Target User Info is not included in the DCR and service-oriented link established as described in TS 23.287 [8] clause 6.3.3.1 is performed.

-	UAS Service Identifier: this identifier is equivalent to V2X Service Type (see TS 23.287 [8]]). It may be preconfigured or derived from the UAV's CAA-Level UAV ID. It is used during the discovery of the C2 peer (UAV/UAV-C) with service-oriented method (e.g., for dynamic UAV-UAV-C pairing).

Editor's note: Further details on discovery are FFS in coordination with SA2.

NOTE 1:	The final choice of identities in the DCR will need to be aligned with SA2.

4.	The UAV and UAV-C may perform a mutual authentication before next step as described in TS 33.536 [6] if the pairing is pre-determined with security credentials (e.g., long term key) pre-configured. This step is skipped if UAV/UAV-C have valid key material (e.g., KNRP and KNRP ID pair).

5.	The UAV and UAV-C establishes the security for the PC5 link as described in TS 33.536 [6]. The UAV and UAV-C establish the security based on the security information (i.e., key material) received from step 2 or key material derived from a prior mutual authentication.

6.	The UAV-C sends Direct Communication Accept over the established link.

7.	The UAV and UAV-C engage in C2 communication over the secure PC5 unicast link.

### 6.4.3	Evaluation

TBD

# 7	Conclusions

Annex <A>:
<Informative annex title for a Technical Report>

Editor’s Note: Delete Annex if not used.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-07 | SA3#107e Adhoc | S3-221607 |  |  |  | Approved skeleton (S3-221512) plus S3-221604, S3-221605 and S3-221610 | 0.1.0 |
| 2022-09 | SA3#108e | S3-222323 |  |  |  | Incorporating S3-221755, S3-222087, S3-222268, S3-222361, S3-22362, S3-222427 and S3-222428. | 0.2.0 |