- 3GPP TSG-SA3 Meeting #109Adhoc-e	draft_S3-230348-r1

- Online, 16 - 20 January 2023	revision of S3-230348

- Source:	CATT

- Title:	pCR to TR33.740 Update Solution28 for removing ENs

Agenda Item:	5.3

# 1	Decision/action requested

Approve the pCR below

# 2	References

# 3		Rationale

Part1：Remove the following EN without adding extra text for the following reasons:

Editor’s Note: The need of two sets of security parameters is FFS.

As SA2 concluded in TR 23.700-33, there are two data sets, one for U2U relay discovery and the other for direct discovery. If only one set security parameters is used for discovery message protection, then we cannot figure out who is authorized as the Source/Target UE and who is authorized as the Relay UE. Figure out the Source/Target UE and the Relay UE is required by both SA3 and SA2. Therefore, two sets of security parameters are required.

Part2：Remove the following EN without adding extra text for the following reasons:

Editor’s Note: need for discovery security policy is FFS.

See the clarification text in the clause of Introduction.

Part3：Remove the following EN for the following reasons:

Editor’s Note: The need of two different discovery codes (ProSe restricted code and RSC) is FFS.

The ProSe restricted code is a natural part of the direct discovery message and RSC is a natural part of the U2U relay discovery message. Otherwise, a message is not a direct discovery message or a U2U relay discovery message. Therefore, we simply remove this EN without adding extra text.

Part4：Remove the following EN through adding a following NOTE:

Editor’s Note: How security protection is done is FFS.

NOTE: How the direct discovery set and U2U discovery set are protected by the direct discovery security parameters and U2U relay discovery security parameters is left to the normative phase.

# 4	Detailed proposal

#### *** BEGIN CHANGES ***

## 6.28	Solution #28: UE-to-UE relay discovery security

### 6.28.1	Introduction

This solution addresses Key Issue #1: Security for UE-to-UE Relay discovery.

In clause of 8.1 of TR 23.700-33 [2], there are the following conclusions about 5G ProSe UE-to-UE Relay discovery:

-	The 5G ProSe UE-to-UE Relay discovery message contains two sets of elements, i.e. direct discovery set(s) and a U2U discovery set.

-	The direct discovery set of elements can be part of the contents of 5G ProSe Direct Discovery message as defined in Rel-17. This includes for example the User Info ID of Source UE and Target UE.

-	The U2U discovery set contains the information to support the discovery of the UE-to-UE relay and extensions of the direct discovery. This includes for example Type of Discovery Message, RSC, User Info ID of the relay, etc.

-	5G ProSe UE-to-UE relay only modifies the U2U set of the elements, and forwards the end-to-end elements during the discovery procedures.

In clause 5.1.3 of this TR, there are the following security requirements for 5G ProSe UE-to-UE Relay discovery:

The 5G System shall provide a means for confidentiality protection, integrity protection and replay protection of discovery messages for UE-to-UE Relay discovery.

The 5G System shall provide a means to protect the privacy sensitive information of source UE and target UE during UE-to-UE Relay discovery procedure.

This solution proposes reusing the existing security mechanism specified in TS 33.503 [6] to protect the direct discovery set and U2U discovery set in U2U relay discovery messages.

The security requirements for U2U relay discovery messages may vary, such as in a U2U relay application, all the UEs can be the role of Source, Target or Relay UE, and all of them have the same security parameters used to protect discovery messages. In this case, no additional security protection may be required for the direct discovery message. In other cases, only confidentiality or integrity protection may be required. In addition, when the UE is in a low power situation, it may want to disable some security functions, such as direct discovery message security, because in this case, communication capability is more important than security. Therefore, the flexibility of controlling the message security of U2U relay discovery should be considered to meet the requirements of various application scenarios. A security policy indicator is introduced into the U2U relay discovery message to control how to handle the security of the direct discovery set to avoid unnecessary security protection in some scenarios.

### 6.28.2	Solution details

- 6.28.2.1	UE-to-UE relay discovery security of Model A

Figure 6.28.2-1: Security procedure of UE-to-UE relay discovery of Model A

0.	Key management function (PCF/5G DDNMF/PKMF) is responsible for provisioning discovery security parameters to the UEs involved in direct discovery and U2U relay discovery. The key management functions used to provision direct discovery security parameters and U2U relay discovery security parameters can be different. Direct discovery security parameters are provisioned to the Source UE (Monitoring UE) and Target UE (Announcing UE). U2U relay discovery security parameters are provisioned to the Source UE, Target UE and Relay UE, and are associated with a RSC. The security related parameters used in U2U relay discovery are the same as those used in direct discovery specified in TS 33.503 [6].

Editor’s Note: The need of two sets of security parameters is FFS.

For U2U relay discovery Model A, there are:

-	Code-Sending Security Parameters and other security related parameters used in direct discovery are provisioned to the Target UE (Announcing UE).

-	Code-Receiving Security Parameters and other security related parameters used in direct discovery are provisioned to the Source UE (Monitoring UE).

-	Code-Sending Security Parameters and other security related parameters used in U2U relay discovery are provisioned to the Target UE (Announcing UE) and Relay UE. These parameters are associated with a RSC.

-	Code-Receiving Security Parameters and other security related parameters used in U2U relay discovery are provisioned to the Source UE (Monitoring UE) and Relay UE. These parameters are associated with a RSC.

NOTE:	More details about how to provision discovery security parameters to the Source UE, Target UE, and Relay UE are not addressed in this solution.

U2U relay discovery security policy can be provisioned or preconfigured in the Source UE. U2U relay discovery security policy specifies what security protection should be applied to the direct discovery message carried in the relay discovery message. Security protection can be confidentiality protection, integrity protection, scrambling protection, or none.

Editor’s Note: need for discovery security policy is FFS.

1.	The Target UE (Announcing UE) generates U2U relay discovery message as follows:

-	Generate direct discovery set (see clause 8.1 of TR 23.700-33 [2]) in the U2U relay discovery message. There are two alternative ways:

-	Alternative 1: Generate the plaintext of direct discovery message sent to the Source UE (Monitoring UE), and then protect it with the direct discovery security parameters by using the procedures defined in 6.1.3.2.3 of TS 33.503 [6] according to the U2U relay discovery security policy. The output of this process is the direct discovery set in the relay discovery message.

-	Alternative 2: Direct discovery set is newly defined for the U2U relay discovery message. It should contain ProSe Restricted Code for direct discovery and can be processed in a way similar to the direct discovery message.

Editor’s Note: The need of two different discovery codes (ProSe restricted code and RSC) is FFS.

-	Generate the plaintext of relay discovery message sent to the Relay UE as specified in clause 8.1 of TR 23.700-33 [2], and then protect it with the relay discovery security parameters associated with the RSC by using the procedures defined in 6.1.3.2.3 of TS 33.503 [6] with the following extensions:

-	RSC indicates which relay discovery security parameters are used to protect the relay discovery message.

-	Message sender indicator indicates this message is generated by the Target UE.

-	U2U Relay discovery security policy indicator indicates how the direct discovery message is protected.

-	Direct discovery set.

NOTE:	How the direct discovery set and U2U discovery set are protected by the direct discovery security parameters and U2U relay discovery security parameters is left to the normative phase.

2.	The Target UE sends the relay discovery message to the Relay UE.

3.	The Relay UE receives the relay discovery message, and knows that the relay discovery message is targeted at it by checking the Message sender indicator which shows the generator is a Target UE, and then perform the following operations:

-	The Relay UE undoes the received U2U relay discovery message with relay discovery security parameters associated with RSC by using the procedures defined in 6.1.3.2.3 of TS 33.503 [6].

-	The Relay UE generates a new plaintext of relay discovery message sent to the Source UE as specified in clause 8.1 of TR 23.700-33 [2], and then protect it with the relay discovery security parameters associated with the RSC by using the procedures defined in 6.1.3.2.3 of TS 33.503 [6] with the following extensions:

-	RSC indicates which relay discovery security parameters are used to protect the relay discovery message.

-	Message sender indicator indicates this message is generated by the Relay UE.

-	U2U Relay discovery security policy indicator obtained from the received relay discovery message.

-	Direct discovery set obtained from the received U2U relay discovery message.

4.	The Relay UE sends the new generated relay discovery message to the Source UE.

5.	The Source UE receives the relay discovery message and knows that the relay discovery message is targeted at it by checking the Message sender indicator, which shows the generator is a Relay UE, and then perform the following operations:

-	Undo the received relay discovery message with the relay discovery security parameters associated with the RSC.

-	If the U2U relay discovery security policy indicator shows that the direct discovery message is protected, undo the direct discovery message or the direct discovery set with the direct discovery security parameters.

- 6.28.2.2	UE-to-UE relay discovery security of Model B

Figure 6.28.2-2: Security procedure of UE-to-UE relay discovery of Model B

0.	Key management function (PCF/5G DDNMF/PKMF) is responsible for provisioning discovery security parameters to the UEs involved in direct discovery and U2U relay discovery. The key management functions used to provision direct discovery security parameters and U2U relay discovery security parameters can be different. Direct discovery security parameters are provisioned to the Source UE and Target UE. U2U relay discovery security parameters are provisioned to the Source UE, Target UE and Relay UE, and are associated with a RSC. The security related parameters used in U2U relay discovery are the same as those used in direct discovery specified in TS 33.503 [6].

Editor’s Note: The need of two sets of security parameters is FFS.

For U2U relay discovery Model B, there are:

-	Code-Sending Security Parameters, Code-Receiving Security Parameters and other security related parameters used in direct discovery are provisioned to the Source UE (Discoverer UE) and Target UE (Discoveree UE).

-	Code-Sending Security Parameters, Code-Receiving Security Parameters and other security related parameters used in U2U relay discovery are provisioned to the Source UE (Discoverer UE), Target UE (Discoveree UE) and Relay UE. These parameters are associated with a RSC.

NOTE:	More details about how to provision discovery security parameters to the Source UE, Target UE, and Relay UE are not addressed in this solution.

U2U relay discovery security policy can be provisioned or preconfigured in the Source UE and Target UE. U2U relay discovery security policy specifies what security protection should be applied to the direct discovery message carried in the relay discovery message. Security protection can be confidentiality protection, integrity protection, scrambling protection, or none.

Editor’s Note: need for discovery security policy is FFS.

1.	The Source UE (Discoverer UE) generates relay discovery message as follows:

-	Generate direct discovery set (see clause 8.1 of TR 23.700-33 [2]) in the U2U relay discovery message. There are two alternative ways:

-	Alternative 1: Generate the plaintext of direct discovery message sent to the Source UE (Monitoring UE), and then protect it with the direct discovery security parameters by using the procedures defined in 6.1.3.2.3 of TS 33.503 [6] according to the U2U relay discovery security policy. The output of this process is the direct discovery set in the relay discovery message.

-	Alternative 2: Direct discovery set is newly defined for the U2U relay discovery message. It should contain ProSe Restricted Code for direct discovery and can be processed in a way similar to the direct discovery message.

Editor’s Note: The need of two different discovery codes (ProSe restricted code and RSC) is FFS.

-	Generate the plaintext of relay discovery message sent to the Relay UE as specified in clause 8.1 of TR 23.700-33 [2], and then protect it with the relay discovery security parameters associated with the RSC by using the procedures defined in 6.1.3.2.3 of TS 33.503 [6] with the following extensions:

-	RSC indicates which relay discovery security parameters are used to protect the relay discovery message.

-	Message sender indicator indicates this message is generated by the Source UE.

-	U2U Relay discovery security policy indicator indicates how the direct discovery message is protected.

-	Direct discovery set.

NOTE:	How the direct discovery set and U2U discovery set are protected by the direct discovery security parameters and U2U relay discovery security parameters is left to the normative phase.

2.	The Source UE sends the relay discovery message to the Relay UE.

3.	The Relay UE receives the relay discovery message, and knows that the relay discovery message is targeted at it by checking the Message sender indicator which shows the generator is a Target UE, and then perform the following operations:

-	The Relay UE undoes the received U2U relay discovery message with relay discovery security parameters associated with RSC by using the procedures defined in 6.1.3.2.3 of TS 33.503 [6].

-	The Relay UE generates a new plaintext of relay discovery message sent to the Source UE as specified in clause 8.1 of TR 23.700-33 [2], and then protect it with the relay discovery security parameters associated with the RSC by using the procedures defined in 6.1.3.2.3 of TS 33.503 [6] with the following extensions:

-	RSC indicates which relay discovery security parameters are used to protect the relay discovery message.

-	Message sender indicator indicates this message is generated by the Relay UE.

-	U2U Relay discovery security policy indicator obtained from the received relay discovery message.

-	Direct discovery set obtained from the received U2U relay discovery message.

4.	The Relay UE sends the new generated relay discovery message to the Target UE.

5.	The Target UE receives the relay discovery message and knows that the relay discovery message is targeted at it by checking the Message sender indicator, which shows the generator is a Relay UE, and then perform the following operations:

-	Undo the received relay discovery message with the relay discovery security parameters associated with the RSC.

-	If the U2U relay discovery security policy indicator shows that the direct discovery message is protected, undo the direct discovery message with the direct discovery security parameters.

6-10.The Target UE sends discovery response message to the Source UE via the Relay UE. The security process is similar to the steps 1-5.

### 6.28.3	Evaluation

TBD

#### *** END OF CHANGES ***