3GPP TSG-SA3 Meeting #109AdHoc-e 	S3-230088

# Electronic meeting, 16 - 20 January 2023

# Source:	Huawei, HiSilicon

# Title:	New solution to KI#1

# Document for:	Approval

Agenda Item:	5.12 FS_eNS_Ph3

# 1	Decision/action requested

Approve the new KI proposal to eNS3 TR33.886

# 2	References

[1]

# 3	Rationale

The contribution proposes a new solution to the key issue 1.

# 4	Detailed proposal

pCR

***  BEGINNING OF 1st CHANGES  ***

## 5.Y	Key Issue #Y: protecting slice-aware SoR information

### 5.Y.1	Introduction

This solution addresses the key issue 1 providing VPLMN slice information to roaming UE.

The solution provides integrity protection to the slice-aware Steering of Roaming (SoR) information sent to the roaming UE, adapting from the solution in TS33.501 [4] and the Annex C in TS23.122[x].

### 5.Y.2	Solution details

The steps of the solution are described as follows, with reference to figure 5.Y.2-1.

1) The UDM decides to send the slice-aware SoR Information to the UE, with a list of preferred VPLMN/access technology combinations for each identified slice S-NSSAI.

2-3) The UDM shall get SoR-MAC-IAUSF from the AUSF as described below:

Calculation of a SoR-MAC-IAUSF: the parameter P2 specified in A.17 of TS33.501 [4] shall include the list of preferred VPLMN/access technology combinations for each S-NSSAI identified in step 1.

NOTE:	The inclusion of S-NSSAIs in the calculation of SoR-MAC-IAUSF allows the UE to verify that the received SOR Information is not tampered with or removed by the VPLMN

Other parameters and procedures are as specified in clause 6.14.2.3 of TS33.501 [4]

Figure 5.Y.2-1 Procedure for providing slice-aware SoR information

4-5)	The UDM sends a message Nudm_X to the VPLMN AMF which forwards to the UE. The message shall include the slice-aware SoR information and SoR-MAC-IAUSF as in Step 2. Other parameters and procedures are as specified in TS33.501 [4].

6) On receiving the NAS message with the SoR information from the AMF, the UE shall calculate the SoR-MAC-IAUSF in the same way as the AUSF (as in step 3), and verifies whether it matches the SoR-MAC-IAUSF value received.

7-9) As specified in TS33.501 [4].

### 5.Y.3	Evaluation

This solution addresses the key issue 1, reusing the current SoR mechanism as defined in TS 33.501 [4] and TS23.122 [x] for SoR information delivery.  Specifically, it provides integrity protection to the slice-aware Steering of Roaming (SoR) information sent to the roaming UE, adapted from the solution in TS33.501 [4] and TS23.122[x].  It is applicable to both the registration procedure of roaming UE and a UDM initiated SoR information update procedure.

***	  END OF 1st CHANGES	***

***  BEGINNING OF 2nd CHANGES  ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TR 23.700-41 “Study on enhancement of network slicing; Phase 3”

[4]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[x]	3GPP TS 23.122: " Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

***	END OF 2nd CHANGES	***