3GPP TSG-SA3 Meeting #109AdHoc-e 	S3-230089

# Electronic meeting, 16 - 20 January 2023

# Source:	Huawei, HiSilicon

# Title:	New Key Issue on SoR AF

# Document for:	Approval

Agenda Item:	5.12 FS_eNS_Ph3

# 1	Decision/action requested

Approve to include the PCR for the new key issue in TR33.886

# 2	References

[1]

# 3	Rationale

The contribution proposes to study a new key issue in order not to leak S-NSSAI information outside 3GPP systems.

# 4	Detailed proposal

pCR

***  BEGINNING OF 1st CHANGES  ***

## 4.X	Key Issue #X:

### 4.X.1	Key issue details

It has been concluded in TR23.700-41 [3] that normative work will specify a slice based SoR mechanism to deliver enhanced slice-aware SoR information reusing the current SoR mechanism defined in TS 23.122 [x]. According to the current SoR mechanism, the list of the preferred PLMN/access technology combinations may be generated by a dedicated steering of roaming application function (SOR-AF). However, it is a security requirement that the S-NSSAI cannot be to be sent to an AF if it is deployed outside the 3GPP operator domain (TS 33.501 [4]).

### 4.1.2	Security threats

If an S-NSSAI is sent to a SoR-AF deployed outside the 3GPP operator domain, sensitive information may be leaked out of 3GPP systems.

### 4.1.3	Potential security requirements

The 5G system shall prevent S-NSSAI information leakage to deliver slice-aware SoR information.

***	END OF1st CHANGES	***

***  BEGINNING OF 2nd CHANGES  ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TR 23.700-41 “Study on enhancement of network slicing; Phase 3”

[4]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[x]	3GPP TS 23.122: " Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

***	END OF 2nd CHANGES	***