3GPP TSG-SA3 Meeting #109AdHoc-e	S3-230203

# 16 January - 20 January 2023

# Source:	Intel

# Title:	Conclusion for KI#3

# Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

This contribution proposes a conclusion on KI#3 in TR 33.738.

# 2	References

[1]   3GPP TR 33.738 V0.4.0 "Study on security aspects of enablers for Network Automation for 5G - phase 3"

[2]   3GPP TR 23.700-81 V2.0.0 " Study of Enablers for Network Automation for 5G System (5GS); Phase 3"

# 3	Rationale

This pCR proposes to use the ideas of solutions #1, #2, #7,#10,#11.#12, and #13 for the protection of ML models and authorization of model retrieval to conclude KI#3.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.738.

*****START CHANGE*****

# 7	Conclusions

## 7.X	Conclusion on Key Issue #3 "Security for AI/ML model storage and sharing"

For key issue#3, it is recommended to use the ideas of solutions #1, #2, #7, #10,#11.#12, and #13 for the protection of AI/ML models and authorization of model retrieval:

-	Authorization of the model retrieval at the NRF uses OAuth 2.0 token-based authorization. The NRF uses information stored by the MTLF at the NRF as per the conclusion of KI#5 (‘Enhanced trained ML model sharing) in TR 23.700-81, specifically the Interoperability indicator. The AnLF is required to be registered in the NRF with vendor ID information in its profile

-	Additionally, upon the AnLF's request for Analytics Id, the MTLF performs authorization of the corresponding model retrieval per selected model and then provides the model address and encryption key to NFc.

-	NF consumer(s) (MTLF or AnLF) uses the ADRF ML model retrieval service to retrieve the ML model or URL (where the model is stored) from ADRF. ADRF verifies that the requested AI/ML model can be retrieved by the NF consumer(s) (MTLF or AnLF).

-	During normative work, specify which keys to use for end-to-end protection of ML models, i.e., storage of the AI/ML models in encrypted format ADRF and secure distribution of keys to enable the decryption at the consumer side.

*****END CHANGE*****