3GPP TSG-SA3 Meeting #109AdHoc-e 	S3-230226

# Electronic meeting, 16 - 20 January 2023

# Source:	Ericsson

# Title:	Resolving ENs in solution #13

# Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

It is proposed to approve the pCR to TR 33.739.

# 2	References

[1]	3GPP TR 33.739: "Study on security enhancement of support for edge computing phase 2"

# 3	Rationale

This contribution resolves the following ENs in Solution #13 in TR 33.739 [1].

Editor’s Note: It is FFS whether steps 3.a and 3.b are necessary.

Editor’s Note: Further evaluation is FFS.

# 4	Detailed proposal

Approve the following changes to TR 33.739 [1].

*** Start of Change ***

## 6.13	Solution #13: A solution for authentication of EEC/UE and GPSI verification by EES/ECS

### 6.13.1	Solution overview

This solution addresses the key issue #2.1 where it is stated that EEC/UE authentication is required considering both the roaming and non-roaming cases. EEC authentication means verification of the EEC ID sent from the EEC to the ECS/EES while UE authentication means verification of the GPSI of the UE that hosts the EEC communicating with the ECS/EES.

The solution proposes usage of AKMA for UE and EEC authentication by the ECS/EES. In addition to the AKMA usage for UE authentication, this solution uses AF Specific UE ID retrieval API as an optional solution to authenticate the UE, i.e., to verify the GPSI.

### 6.13.2	Solution details

The procedure flow of the solution is depicted in Figure 6.13.2-1 and the steps are explained below. The solution assumes that there is a mechanism in the UE to authenticate the EEC.

Figure 6.13.2-1: Authentication of EEC and UE by ECS/EES

Step 0. UE and HPLMN run primary authentication and derive KAKMA. UE learns the A-KID and KAKMA.

Step 1. KAF is derived as defined in AKMA procedure. KEEC is derived from KAF and EEC-ID as KEEC = KDF(KAF, EEC-ID). KEEC and E(KAF, EEC-ID) are revealed to the EEC where E(KAF, EEC-ID) is the encryption of EEC-ID under the key KAF.

The EEC ID is authenticated in the UE with a method out of the scope of this solution, e.g., by the operating system running in the UE. Since the KEEC derived from the EEC ID is revealed only to the authenticated EEC having the EEC ID, the EES/ECS will ensure that the EEC is the client having the EEC ID after driving the KEEC from the EEC ID.

If the EEC ID is sent over a secure channel between EEC and EES/ECS, then no additional encryption for it is necessary.

Step 2. The EEC sends a session establishment request to the ECS/EES, including the parameters A-KID, E(KAF, EEC-ID).

Step 3. The EES/ECS request KAF from the network. In this request, ECS/EES sends A-KID and ECS-ID/EES-ID.

Step 4. The network executes the authentication and authorization for the ECS/EES as defined in the AKMA procedure and if the result is successful then derives the KAF such that KAF = KDF (KAKMA, ECS-ID/EES-ID).

Step 5. The network sends KAF, expire time for the key and and optionally GPSI to the ECS/EES.

Step 6. Using the KAF key, the ECS/EES decrypts the encrypted EEC-ID and also derives KEEC such that KEEC = KDF(KAF, EEC-ID).

Step 7. The EEC and the ECS/EES use KEEC in Ua* protocol, instead of KAF.

If the EEC sends GPSI to the ECS/EES, then the ECS/EES verifies the received GPSI using the GPSI received from the network via AKMA or via UE ID API.

### 6.13.3	Solution evaluation

This solution addresses key issue #2.1 by proposing a mechanism for authentication of both the EEC and UE by the ECS/EES. The authentication of the EEC by the ECS/EES relies on the authentication of the EEC by the UE with a method out of the scope of this solution.

This solution doesn’t have any impact on the existing AKMA mechanism.

This solution also proposes two alternative methods for validation of the GPSI, by the ECS/EES, which is optionally sent by the EEC to the ECS/EES. These methods are re-using the existing AKMA and UE ID retrieval API.

*** End of Change ***