3GPP TSG-SA3 Meeting #109AdHoc-e 	S3-230251

# Electronic meeting, 16 - 20 January 2023

# Source:	Nokia, Nokia Shanghai Bell

# Title:	update to KI#3 network slice admission control

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

Approve the KI update to TR33.886

# 2	References

[1]	3GPP TR 23.700-41 “Study on enhancement of network slicing; Phase 3”

[2]	3GPP TR 33.886 "Study on enhanced security for Network Slicing Phase 3"

# 3	Rationale

The contribution proposes to update KI#3 to separate potential threats caused by malicious UE and NFs, in addition add security threats and requirements.

# 4	Detailed proposal

***************  Start of 1st change  ************

## 4.3	Key Issue #3: network slice admission control (NSAC)

### 4.3.1	Key issue details

The network slice admission control (NSAC) issues were studied in Rel-17. It has been agreed in Rel-18 to enhance NSAC features with the following features:

- improved network control of the UE behaviour

- support deploying multiple NSACF

In both cases, better UE admission control is aimed to match the allocated quota. However, potential issues of Denial of service (DoS) attacks to legitimate UEs when the additional features are added to the access control mechanism. The information of actual UE / PDU session usage by a slice, or misinformation provided by malicious UEs or mischievous NFs may not be reflected based on current solutions. For example, a NSACF in a VPLMN updating the number of registered UEs or PDU sessions independently may not provide trusted information to the home NSACF. Another example is when a UE not using a network slice is still counted against quota usage of S-NSSAIs where it is registered. It is notable that an attacker can use legitimate UEs to launch such attacks.

In the TR23.700-41 [3], the issue of how to support network slice admission control (NSAC) involving multiple service areas is being studied, together with multiple solutions accepted. The general assumption is that multiple NSACFs are required, either centralized or distributed. In a roaming scenario, it is assumed that the NSAC may be controlled by an NSACF in the VPLMN or an NSACF in the HPLMN.

However, in a roaming scenario, the information reported by the NSACF in the VPLMN is not verified when it is reported to the HPLMN, i.e. there is no proper home control and a misinformation provided by VPLMN may have negative impact to the slices in other service areas, either in HPLMN or other VPLMN.

The security control in different serving areas/PLMNs could be different. For example. the security measure in some service areas is not as strict as what in other areas, attack surface in PLMN of one region may be higher than other regions, etc. The compromised/malicious NSACFs (for solution#13 in TR23.700-41 [3]) in some high risky serving areas/networks may trigger DoS or other attacks on the home network, e.g., the compromised/malicious (local/distributed) NSACFs in a risky service area may fake the case that the number of registration UEs/PDU sessions is reaching the maximum number, and send Nnsacf_NSAC_NumberUpdate_Request to the Primary NSACF for new quota. The Primary NSACF may allocate more quota to the NSACF in compromised serving area/network while decrease the quota of other “lower load” area. Finally, the service of other serving areas/networks could be impacted as the global maximum number may be exhausted by the compromised/malicious NSACFs. As the attack complexity is relatively low while the availability impact could be high, the risk on the system could be high.

In 5G network, network functions in serving network are not always trusted by HPLMN, and home control was enhanced in 5G security architecture. For example, linking home control to subsequent procedures during/after primary authentication procedures to prevent certain types of fraud from serving network (see 6.1.4 of TS 33.501). If AMF/SMF is compromised, only the service of the attacked area would be impacted. In 3GPP protocol point of view, it can hardly damage the HPLMN and impact service of other areas without involving huge number of “legal” UEs due to better home network control for UE access. However, home control for NSAC procedure is not defined in existing specification. In addition, there’s almost no cost/complexity for the compromised/malicious NSACF to launch (D)DoS attack to HPLMN, while it could cause potential wider service impact.

### 4.3.2	Security threats

### The malicious/compromised NSACF(s) in specific area(s) of a PLMN with low security protection may continuously send fake message primary NSACF to indicate the number of registration UEs/PDU sessions is reaching the maximum number, that may cause the primary NSACF to re-distribute the quotas of maximum number of registration UEs/PDU sessions to NSACFs in serving areas, finally impact the service of other benign serving areas.

### 4.3.3	Potential security requirements

The 5G System shall provide a means for preventing the quota of maximum of registration UEs/PDU sessions of a network slice being exhausted by malicious/compromised NSACF(s).

*************    End of the changes	*********