3GPP TSG-SA3 Meeting #109e-AdHoc	S3-230292

# e-meeting, 16 - 20 January 2023

# Source:	Qualcomm Incorporated

# Title:	Updates on the solution #23

# Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution proposes to update the solution #23 in TR 33.740.

# 2	References

[1]	TS 33.503

[2]	TR 33.740 v0.4.1

# 3	Rationale

This contribution proposes to update the solution #23. In particular, this contribution proposes to add a text to clarify that the provisioning of the restricted code and the discovery security materials for direct discovery set protection is same as the existing provisioning procedure specified in clause 6.1.3.2.2 in TS 33.503 [1].

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR [2].

***** START OF CHANGES *****

## 6.23	Solution #23: Security mechanism for UE-to-UE Relay Model A discovery

### 6.23.1	Introduction

This solution addresses the Key Issue #1.

An Announcement message by the U2U relay includes two sets of elements, i.e., direct discovery set(s) (e.g., the User Info ID of source UE and target UE) and a U2U discovery set (e.g., Type of Discovery Message, RSC, User Info ID of the relay). The 5G ProSe U2U Relay only modifies the elements of the U2U discovery set. The direct discovery set is constructed by the target UE and is only interpreted by the source UE. The U2U Relay can use a single RSC (and the associated security materials) to relay direct discovery sets associated with the multiple ProSe services. This means that the 5G ProSe U2U Relay uses the same security materials to protect the Announcement messages that contain the direct discovery sets associated with different ProSe service. Unless each direct discovery set is protected using the security materials of the corresponding ProSe service, a source UE (or a target UE) that is authorized to use the RSC for U2U discovery can decrypt (and manipulate) any direct discovery sets that are delivered using the same RSC. This poses security threats.

To prevent such security threats, this solution proposes a U2U discovery security mechanism that protects the Announcement message using two sets of security materials. The Announcement message and the direct discovery set(s) are protected using the respective set of security materials.

### 6.23.2	Solution details

Figure 6.23.2-1: Model A discovery

0. This solution consists of two protection mechanisms using two sets of security materials: one for the direct discovery set(s) protection and the other one for the Announcement message protection.

Editor’s Note: The need of two sets of security parameters is FFS.

Editor’s Note: It's FFS whether ProSe services requiring different security materials share the same RSC.

1. Direct discovery set(s) protection by the target UE:

The source UE and target UEs are provisioned with the ProSe restricted code and the discovery security materials associated with the ProSe restricted code as specified in clause 6.1.3.2.2 of TS 33.503 [6].

The target UE protects the direct discovery set(s) using the discovery security materials associated with the ProSe restricted code.

The target UE provides the protected direct discovery set(s) to the U2U Relay.

Editor’s Note: how the target UE provides the protected direct discovery set(s) to the U2U Relay needs to be aligned with the U2U relay Model A discovery procedure defined by SA2.

Editor’s Note: The need of two different discovery codes (ProSe restricted code and RSC) is FFS.

NOTE: the protection mechanisms specified in clause 6.1.3.2.3 of TS 33.503 [6] are reused for the direct discovery set and U2U discovery set protection. The details of how the protection mechanisms in clause 6.1.3.2.3 of TS 33.503 [6] is applied to the U2U discovery message protection will be specified during the normative work.

2. Announcement message protection by U2U Relay:

The U2U Relay and source UE are provisioned with the RSC for a U2U relay service and the discovery security materials associated with the RSC as specified in clause 6.1.3.2.2 of TS 33.503 [6].

The U2U Relay protects an Announcement message using the provisioned discovery security materials associated with the RSC as specified in clause 6.1.3.2.3 of TS 33.503 [6]. The Announcement message contains the direct discovery set(s) provided by the target UEs.

The source UE decrypts and/or verifies the received Announcement message using the discovery security materials associated with the RSC. The source UE extracts the direct discovery set(s) from the Announcement message and decrypts and/or verifies the direct discovery set(s) using the discovery security materials associated with the ProSe restricted code.

### 6.23.3	Evaluation

This solution addresses the Key Issue #1. In particular, this solution supports the scenario where a single RSC is used by a 5G ProSe U2U Relay to relay multiple direct discovery sets associated with different ProSe restricted codes between source UEs and target UEs.

Editor’s Note: Further evaluation is FFS.

***** END OF CHANGES *****