3GPP TSG-SA3 Meeting #110Ad-Hoc-e 	S3-231816r4

- Electronic meeting, Online, 17 - 21 April 2023

- Source:	Nokia, Nokia Shanghai Bell

- Title:	pCR on EN removal multiple AMF scenarios Veriant1

- Document for:	Approval

Agenda Item:	4.14

# 1	Decision/action requested

Approve the changes in clause 4 for inclusion in HNTRA normative work

# 2	References

[1]	HNTRA Draft TS

# 3	Rationale

As mentioned in the draft TS 33501 for HNTRA study with Editor’s note:

Editor’s Note: The handling of multiple AMFs registered to the UDM is FFS.

It is possible that two different AMFs are registered in the UDM for different access. In this case, it is possible that UE may not be reachable in one AMF but be reachable in another AMF. Or, UE is not reachable in both AMFs. Therefore, UDM needs to contact any AMF first to perform reauthentication. If not succeeded in the first AMF due to any reason, then the UDM should contact another AMF to perform the reauthentication.

It is assumed that the AMF will perform reauthentication once UE becomes available there. Once any AMF performs reauthentication, the UDM shall cancel the reauthentication on another AMF.

Let's try to understand this with an example:

If there are multiple AMFs (i.e. AMF1 and AMF2) registered in the UDM, the UDM shall select any AMF (e.g. AMF1) to perform the reauthentication. If reauthentication is not performed within a short time or no success response is received from AMF1, the UDM selects another AMF and triggers reauthentication (Step5-7). Once reauthentication is successful in any AMF (e.g AMF2), the UDM shall send a cancellation request to the other AMF2 to avoid AMF2 performing reauthentication once UE becomes available in AMF2.

Another issue is that when UE attaches to 3GPP access and non-3GPP access over the same AMF. In this case, when UDM requests AMF to perform reauthentication. The AMFs should try to authenticate a UE over first access. If UE is not reachable over the first access, then AMF should try to authenticate UE over another access.

We have submitted two variants of the same solution (another one is available at S3-231817). Any one of them should be selected.

# 4	Detailed proposal

**** START OF CHANGE ****

## 6.1X	Home network triggered primary authentication procedure

### 6.1X.1	General

In case the HN and the SN support the services defined in clause 14.2.x, then the UDM is able to trigger a primary authentication procedure as described in the following clause.

Editor's Note: The impact of HONTRA on the security related service operations in clause 14 is FFS. The above statement wil be updated when the service operations are agreed.

### 6.1X.2	Security mechanisms

The UDM may initiate primary authentication based on events from other NFs, considering local policy into account as well.

Figure 6.1.x.2-1 Home Network triggered primary authentication procedure

0a.[optional] The UDM may be pre-configured with an operator policy in order to determine when to trigger a primary authentication procedure.

A prerequisite for the initiating of Home Network Triggered Primary Authentication is that the UDM already has the information about the AMF serving the UE. Otherwise, the UDM cannot contact any AMF in later steps.

Editor’s Note: Whether a new UDM service is needed is FFS, whether it is needed that a consumer NF (e.g. AUSF) triggers the UDM is FFS.

1. Based on a received event and the local operator policy, if there is no ongoing primary authentication for the UE, and if the UDM determines to trigger the primary authentication, the UDM determines the serving AMF/SEAF of the target UE.

If there are different AMFs registered in the UDM for different access, the UDM shall select one AMF to perform the reauthentication. The UE can first select the AMF corresponding to 3GPP access type, or select the AMF under which the UE is reachable or in the CM-CONNECTED state based on the UE subscription data or UE access and mobility information to perform the reauthentication.

NOTE 1:	The reasons for the UDM determining that the UE needs to be authenticated can be different. For example, the UDM can determine to initiate a primary authentication when the AMF registers the UE upon the Registration procedure during the mobility from EPC or when SoR/UPU counters are about to wrap around, or when required based on local policy. The UDM behaviour is determined by the primary authentication policy which takes into account the support of certain features in the PLMN. For example, if the PLMN does not support the SoR/UPU feature, then a policy to initiate HONTRA upon SoR/UPU counter wrap around will not be required.

2. The UDM sends an authentication message to the AMF/SEAF with the UE’s SUPI.

Editor’s Note: The name and the type (request/reply, subscribe notify etc) of the interaction between the AMF and UDM is FFS.

3. After receiving the authentication message from the UDM, the AMF/SEAF shall decide whether run the primary authentication procedure based on its own local authentication policy. If the AMF/SEAF cannot run a primary authentication as described in steps 4, the AMF/SEAF sends the authentication response message to the UDM with an acknowledgement which includes failure cause else it is set as success.

Editor’s Note: The serving network has its own authentication policy. Moreover it is possible that the UE cannot be reached. Therefore it is FFS whether it can start a primary authentication right away after receiving the authentication message from the UDM.

Upon receiving a failure from the AMF, the UDM shall check if another AMF is available over the other access. If yes, the UDM shall select another AMF and send an authentication request. Once reauthentication is successful for any AMF, the UDM shall send the authentication cancel request to the other AMF.

Editor’s Note: Details of authentication cancel request is FFS

4. The AMF/SEAF starts the primary authentication procedure as defined in clause 6.2.1 of the present document. If UE is multiple registered in the same PLMN’s serving network, the AMF determines the access type to trigger the primary authentication procedure based on the UE access and mobility information. For example, if UE has different states over multiple access types, the AMF may select the access type over which the UE is in the CM-CONNECTED state. If UE has the same state over multiple access types, the AMF may select the 3GPP access.

The UDM may execute other procedures (e.g. SoR/UPU) depending on the reason that motivated the UDM triggered reauthentication procedure in step 1.

Editor’s Note: The step details such as for aspects covering operator/local configuration/policy usage, UDM logic, or trigger to UDM etc. are FFS.

**** END OF CHANGE ****