3GPP TSG-SA3 Meeting #110e AdHoc 	S3-232014 and S3-231722 merged in draft_S3-231941-r5

- e-meeting, 17 - 21 April 2023

- Source:	Huawei, HiSilicon

- Title:	Additions to the evaluation of tenet 5 on security posture

- Document for:	Approval

Agenda Item:	5.21

# 1	Decision/action requested

Approve the changes in clause 4 for inclusion in TR 33.894 [1].

# 2	References

[1]	3GPP TR 33.894 "Study on applicability of the Zero Trust Security principles in mobile networks"

[2]	NIST Special Publication 800-207 Zero Trust Architecture.

[3]	S3-222517 "Evaluation of tenet 5 on security posture"

[4]	S3-230717 "Alignment of 3GPP’s 5G Security to the fifth NIST Tenet of ZTA"

# 3	Rationale

This contribution proposes changes and further analysis to the evaluation of the fifth tenet of [2] related to security posture. The contribution is based on a previous submission [3]. The contribution borrows some of the observations raised in [4] and that, in our view, are important for the completeness of the evaluation.

# 4	Detailed proposal

It is proposed to approve the following solution for inclusion in TR 33.894 [1].

**** Start of Changes****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	NIST Special Publication 800-207 Zero Trust Architecture.

[3]	3GPP TR 33.738: "Study on security aspects of enablers for network automation for the 5G system Phase 3".

[4]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[5]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[6]	3GPP TS 23.288: "Architecture enhancements for 5G System(5GS) to support network data analytics services".

[x]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3"

[a]	NIST Special Publication 800-92 Guide to Computer Security Log Management.

**** Next Changes****

### 5.1.4	Tenet #5: Maintain the integrity and security posture of all owned and associated assets

#### 5.1.4.1	Description

According to tenet 5 in [2], "The enterprise monitors and measures the integrity and security posture of all owned and associated assets". Further description in [2] shows that this tenet majorly covers operational security and evaluation of the asset’s security posture during evaluation of resource request. The tenet focuses on the posture of all resources and devices. In the context of this study, resources and devices can be assimilated to 5G System consisting of RAN, Core  and UEs ,whereas the RAN and UE being out of scope. In general, for 5G System entities the tenet recommends processes in place in order to ensure that the security best practices and guidelines are followed as well as a robust security monitoring and reporting solution in place.

All the associated assets connected to the network infrastructure are continuously monitored to ensure that they remain configured in a state that is known to be legitimate and secure. Therefore, the security challenge "Trust Nothing and Verify Everything" is considered.

In the 5G core network, this principle refers to the data that can be collected from the NFs that can be used to perform threat assessment as part of continuous security monitoring and trust evaluation.

5.1.4.2	Relevant security mechanisms

The mechanisms specified in TS 23.288 [6] pertaining to data collection from NFs, e.g., clause 6.2.2 and analytics, e.g., clause 6.3 to 6.7 could be considered relevant.

There is currently no explicit standardized security monitoring within NWDAF or in other NF.

The NWDAF defined in 3GPP TS 29.520 [x] is used to preform network analytics on data collected from user equipment, network functions, and operations, administration, and maintenance (OAM) systems, etc.

Monitoring of the integrity and security posture of the 5G Core, that also take into account other factors of the infrastructure, already exists today in operators network infrastructure. These monitoring systems are out of scope of the 3GPP specifications.

5.1.4.3	Evaluation

In general, the tenet touches upon two aspects. The first one is related to operation security. In this regard, it is expected that the proper security practices and guidelines are followed during deployment and operations in order to detect and mitigate vulnerabilities. This includes as described in [2] regular updates, security patches, and mitigation plans should there be a breach, etc. The enforcement of such practices depends heavily on the implementation and deployment context, e.g., infrastructure, enterprise network, etc. Therefore, it does not warrant the development of standard solutions. Nevertheless, it is worth mentioning that the 3GPP security standards in general are continuously evolving. In fact, vulnerabilities in the security mechanisms revealed either by individual 3GPP members or through one of the established CVD programs are always discussed and remediated whenever deemed necessary.

The second aspect is related to monitoring. In this regard, TS 23.288 [6] provides a framework for data collection services that can further enhance whatever security monitoring solution an operator has. For NF monitoring, the framework of [6] includes analytics for performance monitoring in clause 6.6 and load monitoring in clause 6.5. Such data could be used for example to evaluate the state of the NF and whether it is behaving normally. However, the framework in [6] relies on the NWDAF, a 5G Core NF, to leverage such services. From a higher level perspective taking into account the enterprise as a whole with the 5G Core being one part of it, such relevant information exposed by the 5G Core NFs (if any) directly or indirectly via the NWDAF is very likely to be used by an entity outside the 5G Core itself. This could be the operator security monitoring solution or whatever current proprietary solutions being used to achieve the same goals. Overall, this does not warrant standardizing a 5G Core-specific security monitoring procedure.

NWDAF supports data collection and network monitoring for general aspects related to NF load, performance etc. However, the NWDAF is a Network Function in the 5G Service-Based Architecture itself. According to NIST SP 800-92 [a], sections 2.3.2 and 5.1.3, as well as similar guidances on security logs like, it is important to separate and isolate security logs, So, a security monitoring function can be outside the SBA and the security monitoring function itself would be mostly proprietary.

It is worth noticing that on the one hand, such information could include data like network traffic or logs that is not accessible at the SBA layer. This type of data is not specific to the 5G system itself and highly dependent on the deployment, e.g., platforms and technologies used. Therefore, for data collection and sharing , care must be taken in order to follow the security best practices such as the guidelines of NIST SP 800-92 [a] on security logs.

Therefore, based on this analysis, it is worth investigating whether there is any additional information that could be exposed by the 5G Core NFs for monitoring purposes. In the event of that this study determines that strengthening of the external to 3GPP security monitoring is needed, with not yet specified data collection, this information needs to be well defined and explicitly specified to allow for interoperability and secure operation of installed base.

Besides this no further actions are required.

**** End of Changes****