3GPP TSG-SA3 Meeting #110Ad-Hoc-e 	draft_S3-232006-r1

- Electronic meeting, Online, 17 - 21 April 2023

- Source:	BUPT, OPPO

- Title:	New solution on Protecting Ranging/SL Positioning discovery out of network coverage

- Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

It is requested to approve the proposal on Protecting Ranging/SL Positioning discovery out of network coverage.

# 2	References

[1]	3GPP TR 33.893, Study on Security Aspects of Ranging Based Services and Sidelink Positioning

[2]	3GPP TR 23.700-86, Study on Architecture Enhancement to support Ranging based services and sidelink positioning.

[3]	3GPP TS 33.503, Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)

[4]	3GPP TS 22.855, Study on ranging-based services

[5]	3GPP TR 38.845, Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases

# 3	Rationale

As per TR 23.700-86[2], Ranging/SL Positioning services support both in-network coverage, partial-of-network coverage, and out-of-network-coverage. The basic direct discovery security defined for ProSe in TS 33.503 [3] only considers the UEs are in network coverage. In Ranging/SL Positioning services, there are use cases [4][5] in which the UEs can be determined before using the Ranging/SL Positioning services, e.g., public safety, etc. By pre-provisioning shared root key materials for UEs that are to use Ranging/SL Positioning discovery services, UEs are able to discover each other when there is no network coverage.

This pCR addresses Key Issue #3 for out-of-coverage UEs to securely perform Ranging/SL Positioning direct discovery.

# 4	Detailed proposal

SA3 is kindly requested to approve the below pCR to TR 33.893[1].

**** START OF CHANGE ****

- 6.X	Solution #X: Protecting Ranging/SL Positioning discovery out of network coverage

- 6.X.1	Introduction

The solution addresses Key Issue #3: Protection of discovery procedure, in particular, protects Ranging/SL Positioning discovery when UEs are out of network coverage.

In Ranging/SL Positioning services, there are use cases in which the UEs can be determined before using the Ranging/SL Positioning services, e.g., public safety, etc. By pre-provisioning shared root key materials for UEs that are to use Ranging/SL Positioning discovery services, UEs are able to discover each other when there is no network coverage. The shared root key materials may be pre-provisioned to the UEs via user settings at the application layer or pre-configured by the 3GPP network or Ranging/SL Positioning application server before the discovery procedure. Discovery request authorization is implicit, i.e., the UEs which are provided the shared root key materials are considered authorized UEs and can initiate discovery requests.

The UEs which want to discover others and also want to be discovered use the root key KRSLP to derive the discovery request MIC code, and this code is used to determine whether the discovery is matched or not as well as for integrity protection.  In TS 33.503 Prose discovery security [6], the 5G network provides CURRENT_TIME to ensure the UE’s local UTC time counter is close to true time to anti-replay attacks. The proposed solution uses a random-nonce-based challenge-response mechanism to anti-replay attacks, since it may be difficult to ensure the UE’s local time is not fooled when there is no 5G coverage. Furthermore, the root key can also be used for other purposes, such as Ranging/SL Positioning direct unicast communications.

The solution only works for ProSe-capable UEs.

- 6.X.2	Solution details

Figure 6.X.2-1: Ranging/SL Positioning discovery protection procedure when UEs are out of coverage

- The UEs are pre-provisioned with a shared root key KRSLP and associated valid number of the root key usage times for the specific Ranging/SL Positioning service. The shared root key materials can be pre-provisioned to the UEs via user settings at the application layer or pre-configured by the 3GPP network or Ranging/SL Positioning application server before the discovery procedure. The Ranging/SL Positioning service is identified by Ranging/SL Positioning service code, which is assumed known to UEs prior to the discovery procedure. It can be understood as the real-time Restricted Code and Security Parameters generation process by 5G DDNMF/PKMF in TS 33.503 Security for 5G ProSe Discovery [6] are made as pre-provisioning in this solution.

Note 1: If UEs will use more than Ranging/SL Positioning services when they are out of coverage, UEs are pre-provisioned with independent root keys for each Ranging/SL Positioning service. The root key associated with a specific Ranging/SL Positioning service can be identified by Ranging/SL Positioning service code.

Note 2: The root key KRSLP lifetime is implemented by the valid number of the root key usage times since trusted synchronous time may be not available. When the root key expires (i.e., the valid number of the root key usage times is exhausted), the root key shall be re-provisioned via certain methods which can be the same as the above-mentioned root key pre-provisioning methods.

- UE A is a UE which wants to discover other UEs nearby and also wants to be discovered by other UEs. UE A first checks the valid number of the root key usage times is not exhausted. Then UE A chooses a random number nonce 1, and a specific ciphering algorithm. UE A derives Ranging/SL Positioning Discovery Confidentiality Key (RSLPDCK), Ranging/SL Positioning Discovery Integrity Key (RSLPDIK) using the shared root key KRSLP, Ranging/SL Positioning service code, and nonce 1, updates the valid number of the root key usage times as the current valid number of the root key usage times minus 1. UE A encrypts the Ranging/SL Positioning service code using the chosen ciphering algorithm and the RSLPDCK to avoid being tracked by unauthorized parties if UE A constantly sends the same cleartext Ranging/SL Positioning service code. UE A may optionally encrypt its role since the cleartext UE A role may also be considered as privacy. Then UE A generates discovery request message integrity check (DREQ MIC) code using RSLDIK, Ranging/SL Positioning service code, chosen ciphering algorithm identifier, UE A role, and nonce 1.

Note 3: How the RSLPDCK, RSLPDIK, and DREQ MIC code is calculated (e.g., using what ciphering algorithm) is left for normative work.

Note 4: UE A and UE B can be Reference UE, Target UE, and/or Located UE in the Ranging/SL Positioning service. The UE’s role may be determined by the application layer before the discovery procedures.

- UE A broadcasts Discovery Request message, including Encrypted Ranging/SL Positioning service code, chosen ciphering algorithm, [Encrypted] UE A role, nonce 1, DREQ MIC code.

- UE B is a UE in the proximity of UE A. UE B listens for the Discovery Request message, checks the valid number of the root key usage times is not exhausted, derives RSLDCK’and RSLDIK’ the same as which in step 1, and updates the valid number of the root key usage times as the valid number of the root key usage times minus 1. UE B decrypts the Ranging/SL Positioning service code and UE A role (if encrypted) using RSLDCK’. UE B checks the received DREQ MIC code according to RSLDIK’, the decrypted Ranging/SL Positioning service code, the received ciphering algorithm identifier, UE A role, and the received nonce 1. If checked, the Discovery Request message is pre-considered matched and goes to step 4, otherwise, disregards this Discovery Request and keeps listening to other Discovery Request messages.

- UE B chooses a random number nonce 2, and sends Discovery Response message, including UE B role (encrypted using RSLDCK’ if needed), nonce 2, and Discovery Resp Message Integrity Check (DRESP MIC) code. The DRESP MIC is generated by RSLDIK’, the Ranging/SL Positioning service code, UE B role, nonce1, nonce2.

Note 5: How the DRESP MIC code is calculated is left for normative work.

- UE A listens for the Discovery Response message. It checks the received DRESP MIC code according to RSLDIK’, the Ranging/SL Positioning service code, the decrypted (if it was encrypted) UE B role, nonce1, and the received nonce2. If checked, the Discovery Response message is considered fresh and integral, and goes to step 6, otherwise, disregards this Discovery Response and keeps listening to other Discovery Response.

- UE A sends Discovery Freshness Proof message, including Discovery Freshness Proof Message Integrity Check (DFP MIC) Code. The DFP MIC code is generated by RSLDIK, the Ranging/SL Positioning service code, and the received nonce2.

Note 6: How the DFP MIC code is calculated is left for normative work. The DREQ MIC code, DRESP MIC code, and DFP MIC code shall be calculated with different input arguments to avoid replay attacks, e.g., replay DRESP MIC code as DFP MIC code in step 6.

- UE B listens for Discovery Freshness Proof Message, and checks the received DFP MIC code according to RSLDIK’, the Ranging/SL Positioning service code, and the nonce2, if checked, the Discovery Request is considered fresh and matched, otherwise, disregards the Discovery Request and keeps listening to other Discovery Request.

### 6.X.3	Evaluation

TBD.