3GPP TSG-SA3 Meeting SA3#110-adhoc-e 	S3-231886

# Electronic meeting, 17 - 21 April 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | draft-CR | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authorization of selection of participant NWDAF instances in the Federated Learning group. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Adding details for authorization of selection of participant NWDAF instances in the Federated Learning group. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The change pertains to the authorization of selecting participant NWDAF instances in the Federated Learning group. The procedure involves registering with NRF, generating an access token, initiating an NF service request, verifying the access token, and finalizing the FL group by selecting NWDAF MTLF (FL Client) from the list received from NRF. The NWDAF MTLF (FL Server) then requests service from the selected NWDAF MTLF (FL Client), authenticates the NF Service Consumer, aggregates the local model updates to generate a global model update, and sends the updated global model to the NWDAF AnLF or MTLF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

# X.9	Authorization of selection of participant NWDAF instances in the Federated Learning group

Editor’s Note: This clause captures the authorization of selection of participant NWDAF instances in the Federated Learning group.

Figure X.9-1: NWDAF AnLF/NWDAF MTLF (FL Server) Authorization to receive services from NWDAF MTLF (FL Server)/NWDAF MTLF (FL Client)

In order to discover an NWDAF containing MTLF with Federated Learning (FL) capability via NRF:

- An NWDAF containing MTLF supporting FL as a server or client shall additionally include FL capability type (i.e., FL server), and analytics ID as per clause 6.2C of TS 23.288[XX] registration procedure.

If the ML model requires federated learning, the NWDAF registers with NRF, and if the NWDAF MTLF has FL server capability, it discovers and selects other NWDAF(s) MTLF as FL Client(s) from NRF. If the NWDAF MTLF doesn't have FL server capability, it discovers and selects the FL server from NRF.

1-3. The NF(NWDAF AnLF or MTLF) Service Consumer shall send a request to the NRF to receive an access token to request NWDAF MTLF (FL Server) services. After verifying, the NRF shall generate an access token that includes NWDAF MTLF (FL Server specific token) and send it to the NF(NWDAF ANLF OR MTLF) Service Consumer.

4. The NF(NWDAF AnLF OR MTLF) Service Consumer shall initiate an NF service request to the NWDAF MTLF (FL Server), including the access_token_nwdaf. The NF(NWDAF AnLF OR MTLF) Service Consumer shall also generate a Client Credentials Assertion (CCA) token (CCA_NWDAF) and include it in the request message to authenticate itself towards the NF Service Producers.

5. The NWDAF MTLF (FL Server) shall verify if the access_token_nwdaf is valid and start the FL group.

6. If the NWDAF MTLF (FL Server) determines to start the FL group for analytics id, the NWDAF MTLF (FL Server) shall send a Nnrf_AccessToken_Get request to NRF, including the information to identify the target NF (NWDAF MTLF (FL Client)), the source NF (NWDAF AnLF OR MTLF) Service Consumer, the NF Instance ID of NWDAF MTLF (FL Server), Analytics ID, FL local model training service type, FL group ID, and the CCA_NWDAF provided by the NF(NWDAF AnLF OR MTLF) Service Consumer.

7. The NRF shall check whether the NWDAF MTLF (FL Server) and the NF(NWDAF ANLF OR MTLF) Service Consumer are allowed to access the service provided by the identified NF Service Producers (NWDAF MTLF (FL Client)) for the given Analytics ID included in step 6. NRF shall authenticate both NWDAF MTLF (FL Server) and NWDAF (FL consumer, e.g., AnLF) based on one of the SBA methods described in clause 13.3.1.2 of TS 33.501. NWDAF MTLF (FL Server) MAY include an additional CCA for authentication.

8. After successful verification, the NRF shall generate and provide an access token to the NWDAF MTLF (FL Server). The claims in the token include the NF Instance Id of NRF (issuer), NF Instance Id of the NF Service Consumer (subject), NF type of the NF Service Producer (audience), expected service name(s), (scope), expiration time (expiration), FL group ID, Analytics ID(s), ML model ID(s), and optionally "additional scope" information (allowed resources and allowed actions (service operations) on the resources), with NF(NWDAF AnLF OR MTLF) Service Consumer Instance (subject), to authorize both NF(NWDAF AnLF OR MTLF) Service Consumer (e.g., NWDAF) and NWDAF MTLF (FL Server) to consume the services of NWDAF MTLF (FL Client).

9. The NWDAF MTLF (FL Server) finalizes the FL group by selecting NWDAF MTLF (FL Client) from the list received from NRF.

10. The NWDAF MTLF (FL Server) requests service (i.e., local model updates) from the selected NWDAF MTLF (FL Client), including CCA_NWDAF for authentication of the NF Service Consumer (e.g., NWDAF) by the NF Service Producer(s) (e.g., NWDAF ANLF or MTLF).

11. The selected NWDAF MTLF (FL Client) authenticates the NF Service Consumer (e.g., NWDAF ANLF or MTLF) and verifies the access token, ensuring that the NWDAF MTLF (FL Server) identity, FL group ID, Analytics ID(s), and ML model ID(s) are included as access token additional claims.

12. The NWDAF MTLF (FL Server) aggregates the local model updates to generate a global model update and sends the updated global model to the NWDAF AnLF or MTLF. Rest of the procedure is done as per clause 6.2C of TS 23.288[XX]

*** END CHANGES ***