3GPP TSG-SA3 Meeting #110Ad-Hoc-e 	S3-231888

# Electronic meeting, Online, 17 - 21 April 2023

# Source: 	Huawei, HiSilicon

# Title:	solution to KI#2

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

It is proposed to approve the change described in this document.

# 2	References

[1]

# 3	Rationale

This contribution proposes a soluton to address KI#2.

# 4	Detailed proposal

pCR

***  BEGINNING OF CHANGES  ***

## 5.Y	Solution #Y: support network slice with limited time duration

### 5.Y.1	Introduction

This solution addresses key issue #2.

A network slice may be available only for a limited time duration temporarily or periodically active in the deployment as specified in TS23.501. Furthermore, the temaporty or periodically active slice may only be available to a limited number of UEs, which are authorized to access the temaporty network slice for a limited time duration.

This solution discusses two security aspects, i.e., 1) protection of the capability and the validity time information exchanged between the CN and the UE; 2) UE authorization.

### 5.Y.2	Solution details

### 5.Y.2.1 protection of the capability and the validity time information

During the intial registration, a UE sends its capability of supporting the temprory slice only when the security context is available, i.e. the capability of supporting the temprory slice is not in the clear text IE.

Once the UE is authorized to access the temporary slice, the AMF will include the slice validity time information in the registration accept message. Since this validity time information is delivered in NAS messages, no additional security protection is required.

### 5.Y.2.2 UE authorization

If a UE is not subject to NSSAA, the AMF accepts the UE’s request to a temporary slice based on the UE’s subscription at UDM. The registration procedure is similar to the current registration procedure, except that the AMF needs to make sure the UE’s subscription includes the capability of supporting the temprory slice and the slice has not expired.

If the UE is subject to NSSAA, the AMF initiates the NSSAA procedure with additional validity information included in the authorization results provided by AAA.

### 5.Y.3	Evaluation

The solution allows the network provides validity time information to authroized UE to sync the UE with the network. The validity time information is protected based on the current NAS signalling.

***	END OF CHANGES	***