3GPP TSG-SA3 Meeting #110Ad-Hoc-e 	S3-231926

# Electronic meeting, Online, 17 - 21 April 2023							(revision of S3-yyxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | draftCR | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Living CR of the HONTRA procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | HN_Auth |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This contribtion proposes the necessary requirements for the support and usage of a new home triggered primary authentication procedure that is under the control of the home network. The background is that while the in the 5G System, the home network is in general more involved than in previous generations in the primary authentication procedure, it is still the case that only the visisted network that can directly trigger a new fresh run of the primary authentication. Since the first release of the 5G system several use cases have been identified that would benefit from a home triggered procedure and hence this contribution provides the necessary changes to support such mechanism. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new procedure describing the usage and the message flow of a home network triggered primary authentication procedure as well as the definition of the related new services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for a home network triggered primary authenticationliving doc. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This draftCR's revision history: |  | SA3#110: S3-231477, S3-231478 |  |  |  |  |  |  |  |  |

*** BEGIN of 1st CHANGE ***

6.1.X	Home network triggered primary authentication procedure

6.1.X.1	General

In case the HN and the SN support the services defined in clause 14.2.x, then the UDM is able to trigger a primary authentication procedure as described in the following clause.

Editor's Note: The impact of HONTRA on the security related service operations in clause 14 is FFS. The above statement wil be updated when the service operations are agreed.

6.1.X.2	Security mechanisms

The UDM may initiate primary authentication based on events from other NFs, considering local policy into account as well.

Figure 6.1.x.2-1 Home Network triggered primary authentication procedure

0a.[optional] The UDM may be pre-configured with an operator policy in order to determine when to trigger a primary authentication procedure.

A prerequisite for the initiating of Home Network Triggered Primary Authentication is that the UDM already has the information about the AMF serving the UE. Otherwise, the UDM cannot contact any AMF in later steps.

Editor’s Note: Whether a new UDM service is needed is FFS, whether it is needed that a consumer NF (e.g. AUSF) triggers the UDM is FFS.

1. Based on a received event and the local operator policy, if there is no ongoing primary authentication for the UE, and if the UDM determines to trigger the primary authentication, the UDM determines the serving AMF/SEAF of the target UE.

Editor’s Note: The handling of multiple AMFs registered to the UDM is FFS.

NOTE 1:	The reasons for the UDM determining that the UE needs to be authenticated can be different. For example, the UDM can determine to initiate a primary authentication when the AMF registers the UE upon the Registration procedure during the mobility from EPC or when SoR/UPU counters are about to wrap around, or when required based on local policy. The UDM behaviour is determined by the primary authentication policy which takes into account the support of certain features in the PLMN. For example, if the PLMN does not support the SoR/UPU feature, then a policy to initiate HONTRA upon SoR/UPU counter wrap around will not be required.

2. The UDM sends an authentication message to the AMF/SEAF with the UE’s SUPI.

Editor’s Note: The name and the type (request/reply, subscribe notify etc) of the interaction between the AMF and UDM is FFS.

3. After receiving the authentication message from the UDM, the AMF/SEAF shall decide whether run the primary authentication procedure based on its own local authentication policy. If the AMF/SEAF cannot run a primary authentication as described in steps 4, the AMF/SEAF sends the authentication response message to the UDM with an acknowledgement which includes failure cause else it is set as success.

Editor’s Note: The serving network has its own authentication policy. Moreover it is possible that the UE cannot be reached. Therefore it is FFS whether it can start a primary authentication right away after receiving the authentication message from the UDM.

4. The AMF/SEAF starts the primary authentication procedure as defined in clause 6.2.1 of the present document.

The UDM may execute other procedures (e.g. SoR/UPU) depending on the reason that motivated the UDM triggered reauthentication procedure in step 1.

Editor’s Note: The step details such as for aspects covering operator/local configuration/policy usage, UDM logic, or trigger to UDM etc. are FFS.

*** END of CHANGE ***