3GPP TSG-SA3 Meeting #110 	S3-230613

# Athens, Greece, 20 - 24 February 2023

# Source:	3GPP CT6

Title:	LS on clarification of coding of hexadecimal digits in SUCI NAI

# Document for:	Information, Discussion

Agenda Item:  3

3GPP TSG-CT WG6 Meeting #113		C6-220715

Toulouse, France, 15th – 18th November

Title:	LS on clarification of coding of hexadecimal digits in SUCI NAI

Source:	3GPP CT WG6

To:	3GPP CT WG1, 3GPP CT WG4

Cc:	3GPP SA WG3

Contact person:	Sergi Espi

sergi.espi@gi-de.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

During 3GPP CT WG6#113 meeting, the introduction of test cases to cover the SUCI calculation when non-IMSI SUPI type (NSI or GCI or GLI) is configured in the USIM raised some issues that were discussed.

When the USIM is configured as follows:

- non-IMSI SUPI type. We use NSI in this example.

- Protection scheme different than null-scheme. We use either Profile A or B in this example.

Based on these conditions, the 3GPP CT WG6 understanding of the coding of SUCI is the following:

- As described in TS 24.501, SUCI is provided as part of the 5GS mobile identity information element in the REGISTRATION REQUEST message.

- As described in Table 9.11.3.4.1 of TS 25.401: For SUCI with SUPI format set to "Network specific identifier", the SUCI NAI field contains an NAI constructed as specified in subclause 28.7.3 of TS 23.003:

  - Since the protection scheme is Profile A or B, the Scheme Output shall contain an ECC ephemeral public key, a ciphertext value and a MAC tag value. Each one of these fields are formatted as hexadecimal digits as described in Section 2.2B of TS 23.003.

- As described in Table 9.11.3.4.1 of TS 25.401, the SUCI NAI field is encoded as UTF-8 string. 3GPP CT WG6 understands that for the Profile A/B Scheme Output, each hexadecimal digit is encoded separately as UTF-8 character.

  - Note: In IMSI SUPI Types, the Scheme Output is just coded as hexadecimal digits. It is interesting to see in TS 33.501 that examples used in Annex C.4.3.1 use lowercase and in Annex C.4.4.1 use uppercase.

- As described in TS 31.102, based on Service nº125 of the EFUST of the USIM, the ME or the USIM constructs the SUCI and encodes it as UTF-8 string.

- It is assumed that a hexadecimal digit is a value that takes any of these value ranges: [0-9][A-F][a-f]. The fact that a given hexadecimal digit is in uppercase [A-F] or lowercase [a-f] is irrelevant when it comes to mathematical operations. However, it is relevant in UTF-8 coding as the UTF-8 coding of a hexadecimal digit in uppercase   [A-F] is different than in lowercase [a-f]. This means that a hexadecimal digit may have 2 different UTF-8 coding.

  - Note: The examples used in Annex C.4.3.2 and C.4.4.2 of TS 33.501 use uppercase.

3GPP CT WG6 would like to have confirmation that the understanding described above is correct, and would also like to seek guidance on what is the “right” UTF-8 coding convention of a hexadecimal digit.

# 2	Actions

To 3GPP CT WG1

ACTION: 	3GPP CT WG6 kindly asks 3GPP CT WG1 to confirm 3GPP CT WG6 understanding of the coding of the SUCI as described above.

ACTION: 	3GPP CT WG6 kindly asks 3GPP CT WG1 to provide guidance with regards to the UTF-8 coding of hexadecimal digits in SUCI NAI. Specifically whether it should be uppercase or lowercase.

To 3GPP CT WG4

ACTION: 	Based on the feedback provided by 3GPP CT WG1, 3GPP CT WG6 kindly asks 3GPP CT WG4 to take such feedback into consideration and take proper action.

# 3	Dates of next TSG-CT WG6 meetings

3GPP TSG CT6#114			28 February – 3 March 2023 Athens, Greece