3GPP TSG-SA3 Meeting #110 	S3-230641

# Athens, Greece, 20 - 24 February 2023

# Source:	3GPP SA6

Title:	LS on user consent for UE location sharing

# Document for:	Information, Discussion

Agenda Item:  3

3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230351

e-meeting, 11th – 20th January 2023	(revision of S6-230220)

Title:	LS on user consent for UE location sharing

Response to:	n/a

Release:	3GPP Rel-18

Work Item:	EDGEAPP_Ph2

Source:	SA6

To:	SA3

Cc:	n/a

Contact person:	Walter Featherstone

w_featherstone@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	n/a

# 1	Overall description

SA6 has studied “Key issue #17: Discovery of a common EAS”, as captured in TR 23.700-98 V18.0.0. As part of the study, Solution #29: Discovery of a common EAS has been captured to address this key issue. This solution has been identified as one of five candidate solutions for the EDGEAPP_Ph2 normative work, TS 23.558.

Solution #29 assumes that during an initial phase the application clients (ACs), which run on different UEs and need to be served by a common EAS, exchange the UE locations via application layer signalling so that each AC is aware of its own and all other UE locations in the AC group. It is further assumed that before this exchange occurs, each user has been asked to consent whether they allow the application to use the UE's current location and to share it with other group members (i.e., users of the same application). These UE locations will then be passed from the ACs to the EECs and included by the EECs in the Edge Enable Layer (EEL) signalling for EAS discovery.

SA6 consider the application layer mechanism by which a user consents to share the location of their UE with other group member ACs and the related application layer signalling for sharing the location outside the scope of 3GPP.

Given that in this scenario user consent is requested and granted at the application layer, SA6 would like to ask SA3 for guidance on whether the EEC can implicitly assume that such consent was given, when the EEC receives the locations of other UEs in the group from the AC as part of AC group related information, such that the EEC can send this UE location information via EEL signalling towards the ECS and/or EES. If implicit user consent cannot be assumed, SA3 guidance on obtaining that consent is requested.

# 2	Actions

ACTION:

SA6 kindly requests SA3 guidance on whether an EEC’s EEL signalling can include information it receives from an AC related to other UEs (e.g., UE location information of other AC group members), based on the implicit assumption that user consent was obtained prior to application layer signalling of such information to the AC.

# 3	Dates of next TSG SA WG 6 meetings

SA6#53                 27th February – 3rd March 2023 	Athens, Greece

SA6#54-e              17th April – 26th April 2023 	e-meeting