3GPP TSG-SA3 Meeting #110 	S3-230662

# Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0103 | rev | 0.1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of SYN Flood attack prevention test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Federal Office for Information Security (BSI) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS |  |  |  |  |  | Date: |  |  | 2023-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The vendor could use non-standard/arbitrary measures to prevent SYN Flood attacks. There should be vendor documentation on how SYN Flood attacks are prevented. The existing execution steps do not clearly state on how and when the tester should check service availability. The availability check should include the request of typical service functions and it should be executed while the SYN Flood attack is ongoing. MME IP is stated in the test example. This should be Network Product IP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added vendor documentation describing SYN Flood attack prevention mechanism to pre-conditions. Added vendor documentation check to test steps. Added more precise instructions to last execution step. Added executed SYN Flood attack to expected format of evidence. Added configuration part concerning SYN Flood attack prevention to format of evidence. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A missing vendor documentation of the used SYN Flood attack prevention mechanism leads to bad verifiability. The test steps could be interpreted in a way that the SYN Flood attack prevention is not actually checked while SYN Flood attack is ongoing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.3.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

##### 4.3.3.1.4 	SYN Flood Prevention

Requirement Name: Syn Flood Prevention

Requirement Description:

The network product shall support a mechanism to prevent Syn Flood attacks (e.g. implement the TCP Syn Cookie technique in the TCP stack by setting net.ipv4.tcp_syncookies = 1 in the linux sysctl.conf file). This feature shall be enabled by default.

Test Case:

Test Name: TC_SYN_FLOOD_PREVENTION

Purpose:

Verify that the Network Product supports a Syn Flood Prevention technique.

Procedure and execution steps:

Pre-Conditions:

-	Vendor documentation describing the SYN flood attack prevention mechanism or setting and where to check for them

-	The Network Product is listening on a TCP port one of its interfaces.

-	A network traffic analyser on the network product (e.g. TCPDUMP) or an external traffic analyser directly connected to the network product is available.

-	A host is connected to the Network Product interface and it is equipped with a tool able to reproduce a Syn Flood attack (e.g. nmap or hping)

Execution Steps

1.	The tester verifies the prevention mechanism or setting described in the vendor documentation.

2.	The tester configures the tool to send a huge amount of TCP Syn packets against the Network Product (e.g. hping3 -i <waiting time between each packet> -S -p <TCP port> -c <Number of packets> <Network Product IP>)

3.

- While the SYN Flood attack is ongoing and

- After the SYN Flood attack was executed...

... the tester verifies that the Network Product is still up and running normally, its services are still available and reachable, the service is able to respond to typical service function requests, the memory or CPU usage is not exhausted, there is no crash or deadlock.

the tester verifies that the Network Product is still up and running normally, its services are still available and reachable, the service is able to respond to typical service function requests, the memory or CPU usage is not exhausted, there is no crash or deadlock.

Expected Results:

The Network Product does not become inoperative.

Expected format of evidence:

Executed commands or script used for the SYN flood attack.

Part of the configuration (plaintext or screenshot) showing the prevention mechanism or setting.

A Pass/Fail result provided by the tester.

********** END OF CHANGE **********