3GPP TSG-SA3 Meeting #110 	S3-230684

# Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0099 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of brute force mitigation mechanism verification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Federal Office for Information Security (BSI), Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSCAS_5G |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As currently stated between Test Name and Purpose (for better readability we moved this statement to the pre-conditions), the brute force protection measures stated in the requirement description are not needed to be implemented, but if alternative ones are used by a vendor, they also need to be tested. The use of CAPTCHAs is recommended, but optional. In test case A.3 and B.3, CAPTCHA activation is mandatory for the following steps. We added an if-clause to adapt to the optional CAPTCHA pre-condition. When checking for CAPTCHA functionality, the negative case should cover incorrect input and missing input, not only the latter one. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add pre-condition and execution step to test alternatively measures. Make use of CAPTCHA feature optional, to be in line with requirements. Add check for incorrect CAPTCHA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Due to inconsistencies between test case description and execution steps the task for the tester regarding checks of alternative measures is not clear. Inconsistent demand of CAPTCHA usage. Incorrectly working CAPTCHAs could not be detected properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3.4.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

4.2.3.4.3.3	Protection against brute force and dictionary attacks

Requirement Name: Protection against brute force and dictionary attacks

Requirement Description:

If a password is used as an authentication attribute, a protection against brute force and dictionary attacks that hinder password guessing shall be implemented.

Brute force and dictionary attacks aim to use automated guessing to ascertain passwords for user and machine accounts. Various measures or a combination of these measures can be taken to prevent this.

The most commonly used protection measures are:

1)	Using the timer delay (this delay could be the same or increased depending the operator's policy for each attempt, e.g. double the delay, or 5 minutes delay, or 10 minutes delay) for each newly entered password input following an incorrect entry ("tar pit").

2)	Blocking an account following a specified number of incorrect attempts, refer to 4.2.3.4.5. However it has to be taken into account that this solution needs a process for unlocking and an attacker can force this to deactivate accounts and make them unusable.

3)	Using CAPTCHA to prevent automated attempts (often used for Web applications).

4) 	Using a password blacklist to prevent vulnerable passwords.

NOTE 1: 	Password management and blacklist configuration may be done in a separate node that is different to the node under test, e.g. a SSO server or any other central credential manager.

In order to achieve higher security, it is often meaningful to combine two or more of the measures named here. It is left to the vendor to select appropriate measures.

Above requirements shall be applicable for all passwords used (e.g. application-level, OS-level, etc.). An exception to this requirement is machine accounts.

Security Objective references: tba.

Test case:

Test Name: TC_PROTECT_AGAINST_BRUTE_FORCE_AND_DICTIONARY_ATTACKS

Purpose:

To ensure that the system uses a mechanism with adequate protection against brute force and dictionary attacks

To check whether system follows commonly used preventive measures which are mentioned below.

1.	Using the timer delay (e.g. doubling wait times after every incorrect attempt, or 5 minutes delay, or 10 minutes delay) after each incorrect password input ("tar pit").

2.	Blocking an account following a specified number of incorrect attempts (typically 5). However administrator has to keep in account that this solution needs a process for unlocking and an attacker can utilize this process to deactivate the accounts and make them unusable.

3.	Using CAPTCHA to prevent automated attempts (often used for Web interface).

4.	Using a password blacklist to prevent vulnerable passwords.

Procedure and execution steps:

Pre-Conditions:

This test applies only when the most commonly used protection measures used in the requirement are implemented. If they are not implemented, then the vendor documentation needs to provide alternative measures and the tester needs to develop suitable tests for these alternative measures. Since a vendor is free to select appropriate measures, only the vendor selected measures are to be tested.

1.	The password policy management of the network product is configured to use the timer delay after each incorrect password input.

2.	The password policy management is configured to block an account following a specified number of incorrect password attempts (typically 5).

3.	The web interface should be configured with CAPTCHA feature to prevent automated attempts.

4.	CAPTCHA feature is optional and test is done only if implemented.

5.	A password blacklist is configured by the tester. At least one blacklisted password (a password that meets the complexity criteria but is blacklisted) is documented.

NOTE 2: 	Password management and blacklist configuration may be done in a separate node that is different to the node under test, e.g. a SSO server or any other central credential manager.

6.	Tester has valid credentials as an authorized user.

7. 	If the recommended protection measures mentioned in the Requirement Description are not implemented in the Network Product, the vendor provides a documentation describing the alternative measures that are implemented instead.

Execution Steps

Execute the following steps:

A.	Positive Test

Case 1:

Test case to use the timer delay after each incorrect password input is covered in clause 4.2.3.4.5.

Case 2:

Test case to block an account following a specified number of incorrect attempts is covered in clause 4.2.3.4.5.

Case 3:

If the network product's login web interface is configured with a CAPTCHA feature, the tester enters the valid password and correct CAPTCHA.

Case 4:

If the recommended protection measures mentioned in the Requirement Description are not implemented in the Network Product, the tester checks if the alternative measures described in the vendor provided documentation are meaningful and develops suitable test cases to verify their correct implementation.

In some cases the network product class can have two or more of the attack prevention methods available, which are a combination of Cases 1-3. In such cases the tester will need to run a combination of these test cases.

B.	Negative Test

Case 1:

Test case to use the timer delay after each incorrect password input is covered in clause 4.2.3.4.5.

Case 2:

Test case to block an account following a specified number of incorrect attempts is covered in clause 4.2.3.4.5.

Case 3:

If the network product's login web interface is configured with a CAPTCHA feature, the tester enters the valid password without and with incorrect CAPTCHA.

Case 4:

1.	The tester tries to change the password to the blacklisted password.

Expected Results:

A.	Positive Test

Case 1:

Expected result for the test case to use the timer delay after each incorrect password input is covered in clause 4.2.3.4.5.

Case 2:

Expected result for the test case to block an account following a specified number of incorrect attempts is covered in clause 4.2.3.4.5.

Case 3:

Tester can login only after entering the correct password and CAPTCHA.

Case 4:

The tester assesses the alternative measures for brute force and dictionary attack mitigation as meaningful and all developed test cases can be completed successfully.

B.	Negative Test

Case 1:

Expected result for the use the timer delay after each incorrect password input is covered in clause 4.2.3.4.5.

Case 2:

Expected result for the test case to block an account following a specified number of incorrect attempts is covered in clause 4.2.3.4.5.

Case 3:

Tester cannot successfully log in to the network product.

Case 4:

Tester cannot successfully change the password to the blacklisted password.

Expected format of evidence:

Evidence suitable for the interface, e.g. screenshot containing the operation result.

********** END OF CHANGE **********