3GPP TSG-SA3 Meeting #110 	S3-230686

# Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0101 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of service reachability restriction verification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Federal Office for Information Security (BSI), Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSCAS_5G |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Fixed a typo in the Test Name. The current execution steps only test the positive case on one interface and the negative case on another interface. The test steps should be formulated in a way that the checks on service reachability and non-reachability are exhaustive for all vendor-stated interface-service combinations. For verifiability and reproducibility reasons, the parameters and settings used in the test execution should be provided as test evidences. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Make the test execution more generic for a variable number of network interfaces and checking for service reachability on all interfaces. Add evidence for port scanner name and version and make configuration evidence more specific. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the network product has more than 2 interfaces, not all of them would be checked. Not all service – interface mappings would be checked. Insufficient verifiability and reproducibility due to imprecise evidences. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

#### 4.3.2.2	Restricted reachability of services

Requirement Name: The network product shall restrict the reachability of services

Requirement Description:

The network product shall restrict the reachability of services so that they can only be reached on interfaces where their usage is required. On interfaces were services are active, the reachability should be limited to legitimate communication peers. This limitation shall be realized on the network product itself (without measures (e.g. firewall) at network side) according to the requirement detailed in clause 4.2.6.2.1 Packet Filtering.

Example:	Administrative services (e.g. SSH, HTTPS, RDP) shall be restricted to interfaces in the management network to support separation of management traffic from user traffic.

Test Case:

Test Name: TC_RESTRICTED_ REACHABILITY _OF_SERVICES

Purpose:

To verify that it is possible to bind the services only to the interfaces from which they are expected to be reachable.

Note: 	The test case developed for the requirement " 4.2.6.2.1 Packet Filtering" implicitly verifies that the network product permits to limit the reachability of the services only to legitimate communication peers,

Procedure and execution steps:

Pre-Conditions:

-	The vendor shall declare, in the documentation accompanying the network product if the network product supports the capability to restrict services reachability to only the nodes authorized to access them. In this case, the vendor shall detail how this capability can be configured.

-	A list of all required network protocols and services containing at least the following information shall be included in the documentation accompanying the Network Product:

-	protocol handlers and services needed for the operation of network product;

-	their open ports and associated services;

-	the configuration options;

-	and a description of their purposes.

-	The network product is configured such that the required network protocols and services (as described in the network product documentation) are setup and each service is bound to an IP address of a specific network interface (e.g. IP1 which is the ip address of if1). Configuration may occur automatically during the initialization phase of the network product or manually as defined in the network product administration documentation.

-	The network product shall have at least two interfaces enabled, if1 and if2 respectively configured with IP Address IP1 and IP2.

-	The tester has administrative privileges.

-	A tester machine equipped with a network port scanner tool is available.

Execution Steps

For every available interface if_n:

1.	The tester runs a network port scanner (e.g. nmap) or uses local network interface information on if_n and verifies that the configured services (according to the vendor documentation)  are open/reachable.

2.	The tester runs a network port scanner (e.g. nmap) or uses local network interface information on all other available interfaces (except if_n) and verifies that the services configured for if_n are not open/reachable.

Expected Results:

Services can be enabled on per-interface basis.

Expected format of evidence:

A testing report provided by the testing agency which will consist of the following information:

-	The network product configuration showing the mapping between interfaces and configured service

-	Pcap files

-	Screenshot

-	Software name and version of the used port scanner, log of the executed commands

-	Network port scanner results (e.g. files containing this results)

-	Test result (Passed or not)

********** END OF CHANGE **********