3GPP TSG-SA3 Meeting #109 	S3-230703

# Athens,Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1530 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | HNTRA procedure for UPU wrap around case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | HN_Auth |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As agreed in the conclusion of TR 33.741, HNTRA procedure is to be defined for different use cases. It is recommended to specify a new Home Network Triggered Authentication procedure for which the UDM is the central point that decides when to trigger the procedure, and its behaviour is controlled by the Home Network. That means that the UDM triggers interaction with the serving network to request the execution of primary authentication. So, we are adding it for UPU wrap around case in this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 6.15.2.2 is updated for UPU wrap around case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with the TR conclusion |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.15.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************ START OF CHANGES

#### 6.15.2.2	UE Parameters Update Counter

The AUSF and the UE shall associate a 16-bit counter, CounterUPU, with the key KAUSF.

The UE shall initialize the CounterUPU to 0x00 0x00 when the newly derived KAUSF is stored (see clause 6.2.2.2). The UE shall store the UPU counter . If the USIM supports both 5G parameters storage and 5G parameters extended storage, then CounterUPU shall be stored in the USIM. Otherwise, CounterUPU shall be stored in the non-volatile memory of the ME.

To generate the UPU-MAC-IAUSF, the AUSF shall use the CounterUPU. The CounterUPU shall be incremented by the AUSF for every new computation of the UPU-MAC-IAUSF. The CounterUPU is used as freshness input into UPU-MAC-IAUSF and UPU-MAC-IUE derivations as described in the Annex A.19 and Annex A.20 respectively, to mitigate the replay attack. The AUSF shall send the value of the CounterUPU (used to generate the UPU-MAC-IAUSF) along with the UPU-MAC-IAUSF to the UE. The UE shall only accept CounterUPU value that is greater than stored CounterUPU value. The UE shall update the stored CounterUPU with the received CounterUPU, only if the verification of the received UPU-MAC-IAUSF is successful. The UE shall use the CounterUPU received from the UDM, when deriving the UPU-MAC-IUE for the UE Parameters Upadate Data acknowledgement.

The AUSF and the UE shall maintain the CounterUPU for lifetime of the KAUSF.

The AUSF that supports the UE parameters update using control plane procedure shall initialize the CounterUPU to 0x00 0x01 when the newly derived KAUSF is stored (see clause 6.2.2.1). The AUSF shall set the CounterUPU to 0x00 0x02 after the first calculated UPU-MAC-IAUSF, and monotonically increment it for each additional calculated UPU-MAC-IAUSF. The UPU Counter value of 0x00 0x00 shall not be used to calculate the UPU-MAC-IAUSF and UPU-MAC-IUE.

The AUSF shall suspend the UE Parameters Update protection service for the UE, if the CounterUPU associated with the KAUSF of the UE, is about to wrap around. When a fresh KAUSF is generated for the UE, the CounterUPU at the AUSF is reset to 0x00 0x01 as defined above and the AUSF shall resume theUE Parameters Update protection service for the UE.  Alternatively, when UDM requests AUSF for UPU protection via the Nausf_UPUProtection procedure, the AUSF may detect a UPU wrap around case in advance considering the maximum value of the counters is fixed at 65536 and shall not suspend the UPU protection service for the UE. Please refer to Figure 6.15.2.2-1 for more details.

.

Figure 6.15.2.2-1: Procedure for reauthentication during the UPU counter wrap around

1.	The UE is authenticated and registered at the 5GC.

2.	The UDM decides to perform the UPU procedure. Therefore, the UDM invokes the Nausf_UPUProtection procedure. The AUSF tries to increment the CounterUPU and figures out that the CounterUPU is about to wrap around or will reach the max value at the next trigger. Therefore, AUSF shall provide an indication of "Counter CounterUPU Reaching Max value" to UDM along with the successful result of the Nausf_UPUProtection procedure. This alerts the UDM that the current UPU procedure is fine, but the next follow-up UPU procedure update will cause a counter's wraparound failure if authentication is not performed. The UDM shall store the received indication.

3. The UDM completes the UPU procedure as defined in section 6.15.2.1.

4. If " CounterUPU Reaching Max value" indication is received in step 2, the UDM sends a notification to the AMF with SUPI and the "reauthentication required" flag set to true.

5. The AMF shall respond to UDM about the confirmation of the message received.

7. Based on the received indication, the AMF shall start the primary reauthentication procedure as defined in section 6.1.2, 6.1.3 and 6.1.4.

8. After Key KAUSF is generated, the CounterUPU shall be reset. Therefore, any further UPU trigger at the UDM will be successful.

************ END OF CHANGES